18CE505

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| February, 2021 | Civil Engineering |  |  |
| Fifth Semester | Water Resources Engineering |  |  |
| Time: Three Hours |  | Maximum : 50 Marks |  |
| Answer ALL Questions from PART-A. |  |  | (1X10 = 10 Marks) |
| Answer ANY FOUR questions from PART-B. |  |  | (4X10=40 Marks) |
| Part - A |  |  |  |

1. Answer all questions                                                                                                                          (1X10=10 Marks)

| a | Define Isohyetal method? | CO1 |
| --- | --- | --- |
| b | What are the factors affecting evaporation? | CO1 |
| c | Distinguish between hyetograph and hydrograph? | CO1 |
| d | Define specific retention and specific capacity? | CO2 |
| e | State the assumptions of Dupuit’s theory? | CO2 |
| f | What is balancing depth? | CO2 |
| g | What is canal lining? What are its advantages? | CO3 |
| h | Write the types of canal outlets? | CO3 |
| i | Discuss the benefits and ill-effects of Irrigation? | CO4 |
| j | Write the relationship between duty and delta for a given base period. | CO4 |

Part - B

| 2.a | List different types of self-recording rain gangs. Explain the working of any one with help of neat sketches? | 5M | CO1 |
| --- | --- | --- | --- |
| 2.b | Discuss various factors affecting run off from a catchment area? | 5M | CO1 |

| 3.a | What is S-hydrograph? How it is constructed? | 5M | CO1 |
| --- | --- | --- | --- |
| 3.b | The hourly ordinates of 2hr unit hydrograph are given below. Derive a 6hr hydrograph for the same catchment. Time(hr); 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15. Discharge (m3/s); 0; 1; 2.7; 5; 8; 9.8; 9; 7.5; 6.3; 5; 4; 2.9; 2.1; 1.3; 0.5; 0. | 5M | CO1 |

| 4.a | Derive the equation for discharge from a well fully penetrated in a confined aquifer? | 5M | CO2 |
| --- | --- | --- | --- |
| 4.b | During a recuperation test, the in an open well was depressed by pumping by 2.5m and it recuperated 1.8m in 80 minutes. Find i) yield from a well of 4m diameter under a depression head of 3m, ii) the diameter of the well to yield 8 litres/sec under a depression head of 2m. | 5M | CO2 |

| 5.a | Explain briefly Kennedy’s design procedure. | 5M | CO2 |
| --- | --- | --- | --- |
| 5.b | Design a regime channel to carry a discharge of 100 cumecs in soil having silt factor of 1.1, using Lacey’s theory. Assume side slope of 0.5 horizontal and 1 vertical. | 5M | CO2 |

| 6.a | Explain causes and remedial measures of water logging? | 5M | CO3 |
| --- | --- | --- | --- |
| 6.b | Design a trapezoidal shaped concrete lined channel to carry a discharge of 250m3/s at a slope of 1 in 6000. The side slopes of the channel are 1.5:1 and limiting depth of 3m is to be maintained. Take the value of N as 0.015. | 5M | CO3 |

P.T.O.

18CE505

| 7.a | What do you understand by a head regulator and cross regulator? | 5M | CO3 |
| --- | --- | --- | --- |
| 7.b | Explain classification of canal falls. | 5M | CO3 |

| 8.a | Explain different types of irrigation? | 5M | CO4 |
| --- | --- | --- | --- |
| 8.b | Explain the working procedure of Drip Irrigation? | 5M | CO4 |

| 9.a | Discuss the factors affecting duty? | 5M | CO4 |
| --- | --- | --- | --- |
| 9.b | Determine the frequency of Irrigation from the following data: Field capacity: 27%, Permanent wilting point: 14%, Dry density of soil: 15 kN/m3, effective depth of root zone: 75cm, Daily consumptive use of water for the crop: 11mm, Readily available moisture: 80% of available moisture. | 5M | CO4 |