18CE503

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| February, 2021 |  |  |  | Civil Engineering |  |  |  |  |
| Fifth Semester |  |  |  | Design of Concrete Structures |  |  |  |  |
| Time: Three Hours |  |  |  |  | Maximum: 50 Marks |  |  |  |
| Answer ANY FIVE from the following. |  |  |  |  |  | (5X10=50 Marks) |  |  |
|  | 1. | a) | Explain transformed area method. |  |  |  | 3M |  |
|  |  | b) | What are the advantages of doubly reinforced beams over singly reinforced beams? |  |  |  | 3M |  |
|  |  | c) | Find the moment of resistance of a singly reinforced beam with width 250mm, overall depth is 400mm and clear cover of 25mm. Use Fe250 and M20 grades. Assume 5-20mm diameter bars as tension reinforcement. Use WSM. |  |  |  | 4M |  |
|  | 2. | a) | Design a doubly-reinforced beam of 350mm x 700mm(effective) which is subjected to a bending moment of 400kN-m. Use Fe415 steel and M20 grade concrete. Assume d’=50mm and fsc=335.3Mpa. |  |  |  | 10M |  |
|  | 3. | a) | A reinforced concrete beam 250mm wide and 400mm effective depth is subjected to ultimate design shear force of 150kN at the critical section near supports. The tensile reinforcement at the section near supports is 0.5%. Design the shear stirrups near the support using LSM. Also design the minimum shear reinforcement at the mid span. Assume M20concrete and Fe250 steel. |  |  |  | 10M |  |
|  | 4. | a) | A reinforced concrete beam is supported on two walls of 750mm thick, spaced at a clear distance of 6m. The beam carries a superimposed load of 9.8kN/m. Design the beam using M20 and Fe415 grades. Take fsc=240N/mm2. |  |  |  | 10M |  |
|  | 5. | a) | A simply supported one-way slab of clear span 3m is supported on masonry walls of thickness 350mm. The slab is used for residential purpose. Design the slab using M30 and Fe415 grades. The live load on slab is 3kN/m2. |  |  |  | 10M |  |
|  | 6. | a) | The slab of a residential building of size 4.3mx6m is simply supported on all four sides on 230mm walls. Assuming an imposed load of 2kN/m2 and load due to finishes of 1kN/m2, design the floor slab. Use M25 concrete and Fe415 steel. Assume mild exposure condition. |  |  |  | 10M |  |
|  | 7. | a) | Design a circular column to carry an axial load of 1000kN. Use M20 concrete and Fe415 steel. |  |  |  | 10M |  |
|  | 8. | a) | A corner column of size 400mm x 400mm located in the lowermost storey of a system of braced frames, is subjected to a factored loads. Pu=1300kN, Mux=190kN-m, Muy=110kN-m. The effective length of the column is 2975mm. Design the reinforcement in the column, assume M25 and Fe415 steel. |  |  |  | 10M |  |
|  | 9. |  | Design a square footing of uniform thickness for an axially loaded square column of 550mmx550mm. The safe bearing capacity of soil is 250kN/m2. Load on column is 950kN. Use M20 concrete and Fe415 steel. |  |  |  | 10M |  |
|  | 10. |  | Design a square footing of uniform thickness for an axially loaded square column of 500mmx500mm. The safe bearing capacity of soil is 220kN/m2. Load on column is 880kN. Use M20 concrete and Fe415 steel. |  |  |  | 10M |  |