18CED21

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| August, 2021 |  |  |  |  | Civil Engineering |  |  |  |
| Sixth Semester |  |  |  |  | Advanced Design of Structures |  |  |  |
| Time: Three Hours |  |  |  |  |  | Maximum: 60 Marks |  |  |
| IS 800:2007, IS 456(2000), IS 3370 part 4 and part 2 is permitted during exam. |  |  |  |  |  |  |  |  |
| Answer ANY ONE question from each unit. |  |  |  |  |  |  | (5X12=60 Marks) |  |
|  | Unit - I |  |  |  |  |  |  |  |
|  | 1. | A cantilever type retaining wall has a 6 m tall stem. It retains earth level with its top. The soil weights 18kN/m3 and has an angle of repose of 30ᵒ. The bearing capacity of the soil is 220kN/m2. Use M25 grade concrete. The coefficient of friction between soil and concrete is 0.55. |  |  |  |  |  | 12M |
|  | (OR) |  |  |  |  |  |  |  |
|  | 2. | Design a pile cap on a group of four friction piles each of 375mm diameter for supporting 500 mm square reinforced concrete column carrying an axial load of 1500 kN at service conditions. The materials used to be used are the concrete grade of M25 and HYSD steel bars of grade 415.. |  |  |  |  |  | 12M |
|  | Unit - II |  |  |  |  |  |  |  |
|  | 3. | A raft foundation is proposed for eight columns carrying loads as shown in figure. Design the foundation. The safe bearing capacity of the soil is 120kN/m2. All columns are 45cm x 45cm. |  |  |  |  |  | 12M |
|  | (OR) |  |  |  |  |  |  |  |
|  | 4. | Design a rectangular elevated water tank of storage capacity is 75,000 liters. Use M30 grade concrete and Fe415 steel. Assume any suitable data. |  |  |  |  |  | 12M |
|  | Unit - III |  |  |  |  |  |  |  |
|  | 5. | Design a gantry girder to be used in an industrial building carrying an overhead travelling crane for the following data. Crane capacity = 250kN, Self – weight of the crane girder excluding trolley = 150kN , Self – weight of trolley, hook, etc. = 50kN, Minimum hook approach = 1.0m, Distance between wheel centres = 3.5m, Centre – to – centre distance between gantry rails = 16m, Self - weight of the rail section = 300N/m, span of gantry 6.5m. Assume any suitable data if required. |  |  |  |  |  | 12M |
|  | (OR) |  |  |  |  |  |  |  |
|  | 6. | Write the procedure in detail to design a gantry girder. |  |  |  |  |  | 12M |
|  | Unit - IV |  |  |  |  |  |  |  |
|  | 7. | Design a welded plate girder for an effective span of 28m and carrying a factored uniformly distributed load of 35 kN/m and factored two concentrated loads of 140 kN acting at 8m from both ends. (a) Design flange and web section; (b) Provide connection between web and flange. Assume any suitable data if required. |  |  |  |  |  | 12M |
|  | (OR) |  |  |  |  |  |  |  |
|  | 18CED21 |  |  |  |  |  |  |  |
|  | 8. | A plate girder is subjected to a maximum factored moment of 4000 kN-m and factored shear force of 600 kN. Find the preliminary sections for the following conditions and cross section, (a) Girder without any stiffener; (b) Girder with end stiffeners only and (c) Girder with end as well as intermediate and transverse stiffeners . |  |  |  |  |  | 12M |
|  | Unit -V |  |  |  |  |  |  |  |
|  | 9. |  | Determine live loads and wind loads for an industrial building which is situated in Vijayawada. The length of the building is 50m, width of the building is 15m, and length of the purlin is 6m. Use G.I sheets for roof covering. Assume any suitable data if required. |  |  |  |  | 12M |
|  | (OR) |  |  |  |  |  |  |  |
|  | 10. |  |  | Design a channel purlin of a roof truss subjected to a maximum bending moment of 10kNm about the major axis and 8kNm about minor axis. Assume that lateral support is provided by the sheeting. |  |  |  | 12M |

BRIEF SCHEME OF EVALUATION

| Q. No. | Description | Marks |
| --- | --- | --- |
| 1 | Dimensions of Retaining wall | 2 M |
|  | Design of stem | 2 M |
|  | Stability Computations | 2 M |
|  | Design of heel slab | 2 M |
|  | Design of toe slab | 2 M |
|  | Stability against overturning, Stability against sliding | 2 M |

| Q. No. | Description | Marks |
| --- | --- | --- |
| 2 | Fixation of width of pile cap based on the given data and permissible end cover to piles and depth of pile cap basd on the thumb rule | 3M |
|  | Check for truss action and design of tension steel | 3M |
|  | Check for shear and Moment action | 3M |
|  | Reinforcement details in pile cap | 3M |

| Q. No. | Description | Marks |
| --- | --- | --- |
| 3 | Fixing the size of the raft based on the loads and safe bearing capacity | 2 M |
|  | Calculate the bending moments, Shear forces in both column strips and middle strips in both longitudinal and transverse direction | 4 M |
|  | Design the depth of the raft slab based on the one way shear, punching shear and bending moment criteria. | 2 M |
|  | Calculate the top reinforcement and bottom reinforcement based on design negative and positive bending moments respectively | 4 M |

| Q. No. | Description | Marks |
| --- | --- | --- |
| 4 | Reading the permissible stresses from IS code and fixing the dimensions of Water tank for the given cpacity | 3M |
|  | Calculations of moments in long and short side walls | 3M |
|  | Design of reinforcement for cantilever moment | 3M |
|  | Design of base slab | 3M |

| Q. No. | Description | Marks |
| --- | --- | --- |
| 5 | Calculation of BM and SF in the gantry girder with the suitable positioning of crane, calculate horizontal bending moment due to surge load and shear force due to horizontal loads | 3 M |
|  | Trial section is selected based on the thumb rules. Calculate Section properties IZZ,, Iyy and ZP for the trial section | 3 M |
|  | Check for moment capacity, check for biaxial bending, check for shear capacity | 3 M |
|  | Check for web buckling and web bearing, check for deflection and design of welds | 3 M |

| Q. No. | Description | Marks |
| --- | --- | --- |
| 6 | The total design procedure if it is explained as mentioned in the Q.No.5 (12 individual steps) | 12M |

| Q. No. | Description | Marks |
| --- | --- | --- |
| 7 | Design flange | 4 M |
|  | Design of web | 4 M |
|  | Design of connection between web and flange. | 4M |

| Q. No. | Description | Marks |
| --- | --- | --- |
| 8 | Preliminary section design for the Girder without any stiffener | 4M |
|  | Preliminary section design for the Girder with end stiffeners only | 4M |
|  | Preliminary section design for the Girder with end as well as intermediate and transverse stiffeners | 4M |

| Q. No. | Description | Marks |
| --- | --- | --- |
| 9 | Calculation of dead loads, live loads and wind loads from from IS code | 6M |
|  | Calculation of Panel dead loads, live loads and wind loads | 6M |

| Q. No. | Description | Marks |
| --- | --- | --- |
| 10 | Calculation of dead loads, live loads and wind loads from the given data and obtain components of these loads parallel and perpendicular to sheeting | 3M |
|  | Try various combination of loads after taking appropriate partial safety factor | 3M |
|  | Calculate maximum bending moment and shear force and check for shear and moment adequacy | 3M |
|  | Check for interaction equation , check for deflection and check for torsional buckling if required | 3M |