14CE804 (C)

Hall Ticket Number:

| IV/IV B.Tech (Regular / Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July, 2021 | Civil Engineering |  |  |
| Eight Semester | Earthquake Resistant Design Of Structures |  |  |
| Time: Three Hours |  | Maximum: 60 Marks |  |
| Answer ALL Questions from PART-A. |  |  | (12X1 = 12 Marks) |
| Answer ANY FOUR questions from PART-B. |  |  | (4X12=48 Marks) |
| Part - A |  |  |  |

| 1. | Answer all questions |  | (12X1=12 Marks) |  |
| --- | --- | --- | --- | --- |
|  | a) | Define natural frequency and amplitude. |  |  |
|  | b) | What is the importance of critical damping? |  |  |
|  | c) | Define focus and epicentre. |  |  |
|  | d) | The qualitative measure of an earthquake is called---------- |  |  |
|  | e) | Define liquefaction & what are the types of liquefaction |  |  |
|  | f) | Distinguish between flow liquefaction and cyclic mobility? |  |  |
|  | g) | List out various load combinations as per IS code |  |  |
|  | h) | Define base shear. |  |  |
|  | i) | What do you mean by soft storey. |  |  |
|  | j) | Why beam column joints are so important in earthquake resistant design of structures |  |  |
|  | k) | Define ductility. |  |  |
|  | l) | Define retrofitting. |  |  |
| Part - B |  |  |  |  |
| 2. | a) | Explain the equation of motion for a free vibration of undamped system having single degree of freedom system. |  | 6M |
|  | b) | A steel beam simply supported has a span of 500mm. the cross section is rectangular 50mm wide and 5mm deep. This is connected at mid span by means of a spring having stiffness of 50N/mm and a load of 1.5kN is applied at the other end of the spring. Determine the natural frequency of the system if E=2x105 N/mm2 and g=9.8m/sec2 |  | 6M |
| 3. | a) | Explain spring action and damping. |  | 5M |
|  | b) | Explain the types of damping in vibrating system. How can evaluate the damping in a vibrating system? |  | 7M |
| 4. | a) | what are strong motion characteristics of earthquake. |  | 6M |
|  | b) | Explain the significance of seismic zoning of India as per IS1893-2002. |  | 6M |
| 5. | a) | List out the effects of liquefaction on built environment. |  | 6M |
|  | b) | Explain in detail the cyclic stress simplified approach for evaluation of liquefaction |  | 6M |
| 6. | The plan and elevation of a three storey RCC school building is shown below. The building is located in seismic zone V. The type of soil encountered is medium stiff and it is proposed to design the building with a special moment resisting frame. The intensity of dead load is 10kN/m2 and the floors are to cater to an imposed load of 3kN/m2. Determine the design seismic loads on the structure by static analysis or lateral load method. |  |  | 12M |
| P.T.O. 14CE804 (C) |  |  |  |  |
| 7. | A framed RCC building of single storey has dimensions of 5mX5mX3.2m high. Columns are located at four corners of the building. The columns are 230 mm X300 mm in size, slab thickness is 120mm. the live load on the floor is 3kN/m2 and the building is on hard rock and falling in seismic zone IV. Analyze a plane frame of the building for dead load and live load combinations. |  |  | 12M |
| 8. | Design a long column as per IS13920 for the following data obtained from the analysis. Axial load = 600kN; Moment = 300kNm; Shear force = 90kN. Assume the size of the column as 450 x 650mm. Use M20 grade concrete and Fe415 grade steel. Show that detailing of the column with a neat sketch. |  |  | 12M |
| 9. | a) | What are the causes for damage to masonry structures? |  | 6M |
|  | b) | List the Key factors influencing Seismic resistance of Masonry Structures |  | 6M |