20EC202

Hall Ticket Number:

| I/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| October, 2021 | Electronics & Communication Engineering |  |  |
| Second Semester | Basic Instrumentation |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer Question No. 1 Compulsorily. |  |  | (14X1 = 14 Marks) |
| Answer ONE question from each Unit. |  |  | (4X14=56 Marks) |

| 1. | a) | Define the terms Precision? | CO1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | b) | Define Static Error? | CO1 |  |  |
|  | c) | List out the Dynamic characteristics of any measurement system? | CO2 |  |  |
|  | d) | What is the difference between AC and DC bridges? | CO2 |  |  |
|  | e) | What are the Sources of Errors? | CO2 |  |  |
|  | f) | What is a delay Line? | CO1 |  |  |
|  | g) | Name any two control front panel controls in CRO? | CO3 |  |  |
|  | h) | How to find unknown frequency using Lissajous figures? | CO3 |  |  |
|  | i) | Write the relation between Stress and Strain? | CO4 |  |  |
|  | j) | Write various transducers used for temperature measurement? | CO4 |  |  |
|  | k) | Write the working principle of CRO. | CO3 |  |  |
|  | l) | What is meant by Seebeck Effect? | CO4 |  |  |
|  | m) | Name the bridges used for measuring resistance. | CO2 |  |  |
|  | n) | Define transducer. | CO4 |  |  |
| Unit - I |  |  |  |  |  |
| 2. | a) | a)Define and explain the following static characteristics of an instrument. i) Accuracy ii) Linearity iii) Sensitivity iv) Resolution | CO1 | 7M |  |
|  | b) | Discuss different types of errors present in an instrument system. | CO1 | 7M |  |
|  |  | Or |  |  |  |
| 3. | a) | Design an Aryton shunt to provide an ammeter with current ranges 0-1mA,10mA, 5mA, and 100mA, using a D'Arsonval movement having an internal resistance of100Ω and full scale deflections of 50µA. | CO1 | 7M |  |
|  | b) | Explain the procedure to extent the range of a ammeter? | CO1 | 7M |  |
| Unit - II |  |  |  |  |  |
| 4. | a) | Explain the operation of Maxwell’s Bridge and derive the condition for balance of a Bridge. | CO2 | 7M |  |
|  | b) | In the case of Hay's Bridge one arm has resistance of 10KΩ .Another arm has a resistance of 6.7KΩ. The third arm 8KΩ in series with a capacitor of 0.5µF. Determine the values of the elements Rx and Lx in the fourth arm. | CO2 | 7M |  |
|  |  | Or |  |  |  |
| 5. | a) | Draw the circuit diagram of Schering’s Bridge and explain the operation of it. | CO2 | 7M |  |
|  | b) | Explain the operation of Q meter circuit with neat diagram. | CO2 | 7M |  |

| Unit - III |  |  |  |  |
| --- | --- | --- | --- | --- |
| 6. | a) | Draw the basic block diagram of oscilloscope and explain the function of each block? | CO3 | 7M |
|  | b) | Explain the measurement of frequency and phase using CRO. | CO3 | 7M |
|  |  | Or |  |  |
| 7. | a) | Explain the concept of Storage oscilloscope along with circuit diagram | CO3 | 7M |
|  | b) | Draw the circuit diagram of Sampling oscilloscope and explain its operation in detail. | CO3 | 7M |
| Unit - IV |  |  |  |  |
| 8. | a) | Derive the expression for Gauge factor of a strain Gauge. | CO4 | 7M |
|  | b) | Demonstrate the construction and principle of working of a linear voltage differential transformer (LVDT). Explain how the magnitude and direction of the displacement of core of an LVDT is detected. Mention the advantages and applications of LVDT. | CO4 | 7M |
|  |  | Or |  |  |
| 9. | a) | Differentiate between active and passive transducers | CO4 | 7M |
|  | b) | Explain the generalized block Schematic of Digital Data Acquisition System and list out its advantages over Analog Data Acquisition System | CO4 | 7M |