14EC802

Hall Ticket Number:

| IV/IV B.Tech (Regular/Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July, 2021 | Electronics and Communication Engineering |  |  |
| Eight Semester | Fiber Optic Communications |  |  |
| Time: Three Hours |  | Maximum: 60 Marks |  |
| Answer ALL Questions from PART-A. |  |  | (12X1 = 12 Marks) |
| Answer ANY FOUR questions from PART-B. |  |  | (4X12=48 Marks) |
| Part - A |  |  |  |

| 1. | Answer all questions |  | (12X1=12 Marks) |  |
| --- | --- | --- | --- | --- |
|  | a) | State Snell’s law. |  |  |
|  | b) | What is V number of a fiber? |  |  |
|  | c) | Give the expression for numerical aperture in graded index fibers. |  |  |
|  | d) | Mention the losses responsible for attenuation in optical fibers. |  |  |
|  | e) | What is the measure of information capacity in optical wave guide? |  |  |
|  | f) | State types of optical couplers. |  |  |
|  | g) | What are the two types of confinements used in LEDs? |  |  |
|  | h) | Define responsivity. |  |  |
|  | i) | A given APD has a quantum efficiency of 75% at a wavelength of 850 nm. If 0.3µW of optical power produces a multiplied photocurrent of 20µA, find the multiplication M. |  |  |
|  | j) | In the context of optical receiver, what is the probability of error? |  |  |
|  | k) | Define Quantum Limit. |  |  |
|  | l) | What is the range of system margin in link power budger? |  |  |
| Part - B |  |  |  |  |
| 2. | a) | Explain the structure and working of optic fiber cable with suitable diagram |  | 6M |
|  | b) | In detail, state any four applications of optical fiber. |  | 6M |
| 3. | a) | Discuss in brief single mode step index fiber and multimode step index fiber |  | 6M |
|  | b) | Define the terms numerical aperture, critical angle and propagation modes in the context of an optical fiber. |  | 6M |
| 4. | a) | What do you understand by intramodal dispersion? Derive the expression for material dispersion. |  | 6M |
|  | b) | Explain the following losses in a fiber (i)Bending losses (ii)Absorption loss |  | 6M |
| 5. | a) | Explain with neat sketches for fiber splicing techniques. |  | 6M |
|  | b) | List out the desirable requirements of a good fiber optic connector? |  | 6M |
| 6. | a) | Draw and explain the structure of Fabry-Perot laser structure. |  | 6M |
|  | b) | Draw and Compare the Construction and Characteristics of PIN and Avalanche Photo Detector |  | 6M |
| 7. | a) | Explain the working principle of a surface and edge emitting LED’s, with the help of a neat structure |  | 6M |
|  | b) | What do you understand by double hetero structure and state its limitations? |  | 6M |
| 8. | a) | Describe any two types of pre-amplifiers used in optical receiver. |  | 6M |
|  | b) | Describe the noise effects on the Optical Receiver Performance |  | 6M |
| 9. | a) | Discuss the possibility of sources of noise in optical receiver. |  | 6M |
|  | b) | Explain the method of measurement setup of OTDR. |  | 6M |