18EE603

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July, 2021 | Electronics and Electronics Engineering |  |  |
| Sixth Semester | Electrical Drives |  |  |
| Time: Three Hours |  | Maximum: 50 Marks |  |
| Answer Question No.1 compulsorily. |  |  | (1X10= 10 Marks) |
| Answer ONE question from each unit. |  |  | (4X10=40 Marks) |

| 1. | Answer all questions |  | (1X10=10 Marks) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | a) | Define an electric drive. |  |  |  |  |
|  | b) | What is meant by active load torque. |  |  |  |  |
|  | c) | Define PLL. |  |  |  |  |
|  | d) | Draw the Speed- torque Characteristics of a DC series motor. |  |  |  |  |
|  | e) | What is plugging? |  |  |  |  |
|  | f) | Compare VSI & CSI fed drives. |  |  |  |  |
|  | g) | What is meant by eddy current drive. |  |  |  |  |
|  | h) | What are the starting methods of synchronous motor? |  |  |  |  |
|  | i) | Write applications of stepper motor. |  |  |  |  |
|  | j) | What is space vector modulation technique? |  |  |  |  |
| UNIT-I |  |  |  |  |  | What is space vector modulation technique? |
| 2. | a) | Draw the block diagram of an electric drive and explain few advantages. |  | 5M |  |  |
|  | b) | Explain the Multi-quadrant operation of an electrical drives with an example. |  | 5M |  |  |
| (OR) |  |  |  |  |  |  |
| 3. | a) | Explain the modes of operation for speed control of drive. |  | 5M |  |  |
|  | b) | A drive has following parameters. J=10 Kg-m2, T=100 ─ 0.1N, N-m, passive load torque Tl=0.05N, N-m Where N is speed in rpm.Initially the drive is operating in steady state. Now it is to be reversed. For this motor characteristic is changed to T= ─ 100 ─ 0.1N. Calculate the time of reversal. |  | 5M |  |  |
| UNIT-II |  |  |  |  |  |  |
| 4. | a) | Explain the transient analysis of separately excited dc motor with armature control. |  | 5M |  |  |
|  | b) | Explain the dynamic braking of separately excited DC motor and draw its speed torque characteristics. |  | 5M |  |  |
| (OR) |  |  |  |  |  |  |
| 5. | a) | Explain the operation of a separately excited dc motor supplied from 1-Φ fully controlled rectifier. Assume Continuous conduction. |  | 5M |  |  |
|  | b) | Explain the operation of a separately excited dc motor supplied from dual converter. |  | 5M |  |  |
| UNIT-III |  |  |  |  |  |  |
| 6. | a) | Obtain the analysis and performance of three phase induction motor when operation from unbalanced source voltages. |  | 5M |  |  |
|  | b) | What is soft start of induction motor? Explain with different circuits. |  | 5M |  |  |
| (OR) |  |  |  |  |  |  |
| 7. | a) | Discuss about the static Scherbius drive in detail. |  | 5M |  |  |
|  | b) | Discuss about the Static Kramer drive with neat sketch. |  | 5M |  |  |
| UNIT-IV |  |  |  |  |  |  |
| 8. | a) | Explain braking methods of synchronous motor. |  | 5M |  |  |
|  | b) | Explain about the switched reluctance motor drives in detail. |  | 5M |  |  |
| (OR) |  |  |  |  |  |  |
| 9. | a) | Explain the brush less dc motor drives and its applications. |  | 5M |  |  |
|  | b) | Briefly explain about the Stepper motor and its advantages |  | 5M |  |  |