18ME303

Hall Ticket Number:

| II/IV B.Tech (Regular/ Supplymentary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| February, 2021 | Mechanical Engineering |  |  |
| Third Semester | Thermodynamics |  |  |
| Time: Three Hours |  | Maximum : 50 Marks |  |
| Answer ALL Questions from PART-A. |  |  | (1X10 = 10 Marks) |
| Answer ANY FOUR questions from PART-B. |  |  | (4X10=40 Marks) |
| Part-A |  |  |  |

1. Answer all questions                                                                                                                          (1X10=10 Marks)

| a. Define Cycle. |  |
| --- | --- |
| b. State Zeroth law of Thermodynamics. |  |
| c. What is an Irreversible process. |  |
| d. Define Heat engine. |  |
| e. What is Kelvin-plank statement. |  |
| f. List out the T-ds equations. |  |
| g. Write down the Helmholtz function. |  |
| h. Define the Pure substance. |  |
| i. What is the use of Mollier chart. |  |
| j. List out the main components in the steam power plant. |  |

Part-B

| 2. a) What is meant by Thermodynamic equilibrium and Classify the systems briefly. |  | 5M |  | M |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. b) A mass of gas is compressed in a quasi-static process from 80 kpa, 0.1 m3 to 0.4 Mpa, 0.03 m3. Assuming that the pressure and volume are related by pvn = constant, find the work done by the gas system. |  | 5 M |  | M |  |  |  |  |
| 3. a) Prove that energy is a property of a system. |  | 5M |  |  |  |  | M |  |
| 3. b) A fluid is confined in a cylinder by a spring loaded, frictionless piston so that the pressure in the fluid is a linear function of the volume p = a + bV. The internal energy of the fluid is given by the following equation U = 34 + 3.15 pV in kJ. If the fluid changes from an initial state of 170 kPa,0.03 m3 to a final state of 400 kPa, 0.06 m3, with no work other than that done on the piston, find the direction and magnitude of the work and heat transfer. |  | 5M |  |  |  |  | M |  |
| 4. a) Derive the steady flow energy equation for Nozzle and Compressor. |  | 5M |  |  | M |  |  |  |
| 4. b) Steam at a 6.87 bar, 2050 C enters in an insulated nozzle with a velocity of 50 m/s. It leaves at a pressure of 1.37 bar and a velocity of 500 m/s. Determine the final enthalpy of steam. |  | 5M |  |  | M |  |  |  |
| 5.a) | Establish the equivalence of Kelvin-Planck and Clausius statements. | 5M |  |  |  |  |  |  |
| 5.b) | A heat engine receives heat at the rate of 1500 kJ/min and gives an output of 8.2 kW. Determine: i) The thermal efficiency ii) The rate of heat rejection. | 5M |  |  |  |  |  |  |
| 6.a) | State and prove the Classius Inequality. | 5M |  |  |  |  |  |  |
| 6.b) | A fluid undergoes a reversible adiabatic compression process from 0.5 Mpa, 0.2 m3 to 0.05 m3 according to the law pv1.3 = constant. Determine the change in enthalpy, internal energy and entropy, heat transfer and the work transfer during the process. | 5M |  |  |  |  |  |  |
| 7.a) | Deduce the Helmholtz and Gibbs functions. | 5M |  |  |  |  |  |  |
| 7.b) | Calculate the decrease in available energy when 25 kg of water at 950 C mix with 35 kg of water at 350 C, the pressure being taken as constant and the temperature of the surroundings being 150 C (Cp of water = 4.2 kJ/kg K). | 5M |  |  |  |  |  |  |
| 8. | Explain the Phase change phenomenon of a pure substance with the help of neat sketches. | 10M |  |  |  |  |  |  |
| 9. | Explain the working principle of Reheat rankine cycle with neat sketch. | 10M |  |  |  |  |  |  |