18ME502

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| February 2021 | Mechanical Engineering |  |  |
| Fifth Semester | I.C. Engines & Gas Turbines |  |  |
| Time: Three Hours |  | Maximum : 50 Marks |  |
| Answer Question No.1 compulsorily. |  |  | (1X10 = 10 Marks) |
| Answer ONE question from each unit. |  |  | (4X10=40 Marks) |

1. Answer all questions                                                                                                                          (1X10=10 Marks)

a) What is the function of Carburetor.

b) What are the Functional divisions of MPFI System..

c) List out the functions of a nozzle.

d) Define Ignition Delay.

e) Define Air/Fuel ratio.

f) What are the main pollutants from an IC engine?

g) What is the major constituent of natural gas?

h) List out the various methods to improve the thermal efficiency of gas turbine plant.

i) What is the main advantage of pulse jet over ramjet engine?

j) What do you mean by propellant with reference to rocket propulsion?

UNIT – I

| 2. a) | Explain the working principle of four stroke SI engine with the help of neat sketches. | 5 M |
| --- | --- | --- |
| 2. b) | Differentiate between the Four stroke and Two stroke engines. | 5 M |

(OR)

| 3. a) | Define carburetion. Explain the working of a simple float type carburetor with the help of a neat sketch. | 5 M |
| --- | --- | --- |
| 3. b) | Explain the working principle of Fuel Injector with neat sketch. | 5 M |

UNIT – II

| 4. a) | Briefly explain the stages of combustion in Compression Ignition engines. | 5 M |
| --- | --- | --- |
| 4. b) | Explain the phenomenon of knock in SI engines. | 5 M |

(OR)

| 5.a) | Define the Following terms. - Brake thermal efficiency ii) Brake specific fuel consumption iii) Mechanical efficiency | 5 M |
| --- | --- | --- |
| 5.b) | The following details were noted in a test on a four-cylinder, four-stroke engine, diameter = 100 mm; stroke = 120 mm; speed of the engine = 1600 rpm; fuel consumption = 0.2 kg/min; calorific value of fuel is 44000 kJ/kg; difference in tension on either side of the brake pulley = 40 kg; brake circumference is 300 cm. If the mechanical efficiency is 80%, calculate (i) brake thermal efficiency (ii) indicated thermal efficiency (iii) indicated mean effective pressure and (iv) brake specific fuel consumption | 5 M |

UNIT – III

| 6.a) | Briefly explain the various liquid fuels used in IC engines as alternative fuels. | 5 M |
| --- | --- | --- |
| 6.b) | Classify the Gaseous fuels and compare the petrol with the liquefied petroleum gas. | 5 M |

(OR)

| 7.a) | Explain the emissions of an engine briefly. | 5 M |
| --- | --- | --- |
| 7.b) | How catalytic converters reduce the engine emissions explain it. | 5 M |

UNIT – IV

| 8.a) | Explain the working principle of open cycle Gas turbine plant with intercooling method and draw the T – s diagram also. | 5 M |
| --- | --- | --- |
| 8.b) | A gas turbine unit receives air at 1 bar and 300 K and compresses it adiabatically to 6.2 bar. The compressor efficiency is 88%. The fuel has a heating value of 44,186 kJ/kg and the fuel air ratio is 0.017 kJ/kg of air. The turbine efficiency is 90%. Calculate the work of turbine and compressor per kg of air compressed and thermal efficiency. Take Cp= 1.147 kJ/kg K and γ = 1.4 for all processes. | 5 M |

(OR)

| 9. | Explain the working of Ramjet and Pulsejet with the help of neat sketches. List out their merits and demerits. | 10 M |
| --- | --- | --- |