| 20CE403 Hall Ticket Number: II/IV B.Tech( Regular) DEGREE EXAMINATION. August,2022; CIVIL Engineering. Fourth Semester; Mechanics of Materials. Time: Three Hours; Maximum:70 Marks. |  |
| --- | --- |
| Answer question 1 compulsory.; (14X1 = 14 Marks). Answer one question from each unit.; (4X14=56 Marks). |  |
|  | 1. | a) | Define Plane stress with an example.. | CO1 |  |  |  |
|  |  | b) | What is meant by a principal stress? | CO1 |  |  |  |
|  |  | c) | Write formula for maximum shear stress on inclined plane | CO1 |  |  |  |
|  |  | d) | State the limitations of Euler’s Column theory. | CO2 |  |  |  |
|  |  | e) | How does a short column fail ? | CO2 |  |  |  |
|  |  | f) | What do you understand by eccentric loads.? | CO2 |  |  |  |
|  |  | g) | What are theories of failures ? | CO2 |  |  |  |
|  |  | h) | Sketch the Core Kernel of a rectangular section | CO3 |  |  |  |
|  |  | i) | Write the governing differential equation for deflection | CO3 |  |  |  |
|  |  | j) | What is conjugate beam method ? | CO3 |  |  |  |
|  |  | k) | State Mohr’s theorem-I. | CO4 |  |  |  |
|  |  | l) | Write the expression for deflection SSB carrying point load at the center. | CO4 |  |  |  |
|  |  | m) | What is meant strain energy ? | CO4 |  |  |  |
|  |  | n) | State the Castigliano’s I theorem. | CO4 |  |  |  |
|  | Unit –I |  |  |  |  |  |  |
|  | 2. |  | An element in plane stress is subjected to stresses σx = -50 MPa, σy = 10 MPa and τxy = -40 MPa. Using Mohr’s circle, determine (a) the stresses acting on an element rotated through an angle θ = 45°, (b) the principal stresses, and (c)the maximum shear stress. Show all results with neat sketches. | CO1 | 14M |  |  |
|  | (OR) |  |  |  |  |  |  |
|  | 3. |  | A short column of rectangular cross-section 80mm by 60mm carries a load 40 Kn at a point 20mm from the longer side and 35mm from the shorter side. Determine the maximum compressive and tensile stress in the section. | CO1 | 14M |  |  |
|  | Unit –II |  |  |  |  |  |  |
|  | 4. | a) | A 2 m long column has a circular cross-section of 10cm diameter. One of the ends of the column is fixed in direction and position and other end is free. Taking FOS as 3, calculate the safe load using: a) Rankine’s formula, take yield stress, c = 560 N/mm2 and, a =1/1600 for pinned ends. b) Euler’s formula, Young’s modulus for C.I. = 1.2 x 105 N/mm2 . | CO2 | 7M |  |  |
|  |  | b) | Explain briefly about Maximum Principal stress theory | CO2 | 7M |  |  |
|  | P.T.O 20CE403 (OR) |  |  |  |  |  |  |
|  | 5. | a) | Derive the expression for crippling load of Euler column fixed at both ends. | CO2 | 7M |  |  |
|  |  | b) | Explain briefly about Maximum shear stress theory | CO2 | 7M |  |  |
|  | Unit –III |  |  |  |  |  |  |
|  | 6. |  | Find the slope at A and D and maximum deflection in the beam by Macaulay’s method. | CO3 | 14M |  |  |
|  | (OR) |  |  |  |  |  |  |
|  | 7. |  | Determine the rotation and deflections at B and C in the cantilever beam shown in figure given below by moment area method. | CO3 | 14M |  |  |
|  | Unit –IV |  |  |  |  |  |  |
|  | 8. |  | Determine the deflection under 120 kN load in the beam shown in Figure. Use Strain Energy Method. | CO4 | 14M |  |  |
|  | (OR) |  |  |  |  |  |  |
|  | 9. | a) | Determine the vertical deflection under load point at A in the simply supported beam subjected to eccentric load ‘10kN’ shown in Figure. Assume constant EI. Use Castgliano’s method. | CO4 | 7M |  |  |
|  |  | b) | Derive the expression for strain energy due to bending | CO4 | 7M |  |  |