18CE504

Hall Ticket Number:

| III/IV B.Tech (Regular / Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2022 | Civil Engineering |  |  |
| Fifth Semester | Design of Steel Structures |  |  |
| Time: Three Hours |  | Maximum: 50 Marks |  |
| Answer ONE question from each unit. |  |  | (5X10=50 Marks) |

| Unit -I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | a) |  | Write a short note on the types of steel sections used for structural steel construction. | CO1 | 5M |
|  | b) |  | Explain in detail about different types of welded joints with neat sketches. | CO1 | 5M |
| (OR) |  |  |  |  |  |
| 2. |  |  | Design a single bolted double cover butt joint to connect boiler plates of thickness 12 mm for maximum efficiency. Use M16 bolts of grade 4.6. Boiler plates are of Fe410 grade. Find the efficiency of the joint. | CO1 | 10M |
| Unit -II |  |  |  |  |  |
| 3. |  |  | Design a single angle section for a tension member of a roof truss to carry a factored tensile force of 225 kN. The effective length of the member is 3 m. Use 20 mm shop bolts of grade 4.6 for the connection. | CO2 | 10M |
| (OR) |  |  |  |  |  |
| 4. |  |  | Design a single angle strut connected to the gusset plate to carry 180 kN factored load. The length of the strut between centre to centre connection is 3 m. | CO2 | 10M |
| Unit -III |  |  |  |  |  |
| 5. |  |  | Design a slab base for a column ISHB 300 @ 577 N/m carrying an axial factored load of 1000 kN. M20 grade concrete is used for the foundation. Provide welded connection between column and base plate. | CO3 | 10M |
| (OR) |  |  |  |  |  |
| 6. |  |  | Design a gusseted base for the column ISMB 300 carrying a factored load of 3600 KN. The column is to be supported on concrete pedestal built with M20 grade concrete. | CO3 | 10M |
| Unit -IV |  |  |  |  |  |
| 7. |  |  | Design abeam of span 5 m carrying a UDL of 20 kN/m, if the compression flange is laterally un supported. | CO4 | 10M |
| (OR) |  |  |  |  |  |
| 8. |  |  | A hall measuring 15m X 6m consists of bems spaced at 3m c/c. RCC slab of 120mm is cast over the beam. The imposed load is 4kn/m2. The beam is supported on 300mm wall. Design one intermediate beam. | CO4 | 10M |
|  |  |  | Unit -V |  |  |
| 9. |  |  | Design a stiffened seat connection to transfer a factored end shear of 400 kN from a beam MB 550 @ 104 kg/m to a column HB 450 @ 87.2 kg/m. | CO5 | 10M |
|  |  |  | (OR) |  |  |
| 10. |  |  | Design a suitable fillet weld for a bracket which is perpendicular to the plane of column flange. Load on the bracket is 100KN acts a distance of 150mm from column flange. Thickness of the bracket plate is 12mm. The column used is ISHB 300. | CO5 | 10M |