18CE506

Hall Ticket Number:

| III/IV B.Tech (Regular / Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2022 | Civil Engineering |  |  |
| Fifth Semester | SOIL MECHANICS |  |  |
| Time: Three Hours |  | Maximum: 50 Marks |  |
| Answer Question No.1 compulsorily. |  |  | (10X1 = 10 Marks) |
| Answer ONE question from each unit. |  |  | (4X10=40 Marks) |

| 1. | a) | Define residual soil. | CO1 |  |
| --- | --- | --- | --- | --- |
|  | b) | Define density and unit weight of soil. | CO1 |  |
|  | c) | Draw Kaolinite clay mineral properties. | CO2 |  |
|  | d) | Distinguish between discharge velocity and seepage velocity. | CO2 |  |
|  | e) | Differentiate between compaction and consolidation. | CO3 |  |
|  | f) | Write short notes on relative compaction. | CO3 |  |
|  | g) | Define permeability of soils and write Darcy’s formula | CO4 |  |
|  | h) | Differentiate between primary and secondary consolidation. | CO4 |  |
|  | i) | Name various heads based flow conditions. | CO4 |  |
|  | j) | Write advantages of Direct shear test. | CO4 |  |
| Unit –I |  |  |  |  |
| 2. | a) | Explain various soil deposits in India. | CO1 | 5M |
|  | b) | Explain different soils based on their formation. | CO1 | 5M |
| (OR) |  |  |  |  |
| 3. | a) | Derive the relation between Dry unit weight, bulk unit weight and Water content. | CO1 | 5M |
|  | b) | A soil has a liquid limit, plastic limit and flow index respectively are 28%, 15% and 12% respectively. Determine liquidity index, plasticity index, consistency index if the natural water content of the soil is 22%. | CO1 | 5M |
| Unit –II |  |  |  |  |
| 4. | a) | Classify Fine grained soils based on Indian soil classification system | CO2 | 5M |
|  | b) | A soil having gravel content 60%, sand content 28% and % fines 12, liquid limit 43%, plastic limit 23%. Classify the soil based on ISCS. Take D60 =1.65 mm;D30 = 0.93 mm; D10 = 0.058 mm. | CO2 | 5M |
| (OR) |  |  |  |  |
| 5. | a) | Explain about Darcy’s law and its validity. | CO2 | 5M |
|  | b) | Determine the average coefficient of permeability when flow is parallel for a deposit consisting of four layers of thickness 3.2 m, 1.8 m, 1.5 m, & 2 m having the coefficient of permeability of 3 x 10-3 mm/sec, 3 x 10-5 mm/sec, 4 x 10-3 mm/sec, 5 x 10-2 mm/sec respectively. Assume the layers are isotrophic. | CO2 | 5M |
| Unit –III |  |  |  |  |
| 6. | a) | Define quick sand condition and derive an expression for critical hydraulic gradient. | CO3 |  |
|  | b) | During a sample collection the soil strata was observed, it consists of a layer of clay up to 1.5 m having density 16 kN/m3 followed by sand deposit 2 m thick having dry density 20 kN/m3 and there is a water table at a depth of 2 m below ground level take wet density of sand as 24 kN/m3.Calculate total stress, effective stress, pore water pressure. | CO3 |  |
| P.T.O. 18CE506 (OR) |  |  |  |  |
| 7. | a) | Explain about factors affecting compaction. | CO3 | 5M |
|  | b) | The following results were obtained from a standard compaction test on a soil sample Water content (%); 4; 8; 11; 14; 17. Mass of wet soil(kg); 1.45; 1.82; 2.02; 1.99; 1.85. Calculate the maximum dry density and optimum water content for the following results obtained from a standard proctor test (SPT) of a sample. Volume of mould is 950 ml. | CO3 | 5M |
| Unit -IV |  |  |  |  |
| 8. | a) | FindExplain about the experimental procedure of consolidation. | CO4 | 5M |
|  | b) | Explain how primary settlement is computed. | CO4 | 5M |
| (OR) |  |  |  |  |
| 9. | a) | Write the merits and demerits of tri axial shear test | CO4 | 5M |
|  | b) | An unconfined compressive test was conducted on undisturbed sample of clay. The sample had a diameter of 37.5 mm and 76 mm long. The load at failure was 50 N and the axial deformation of the sample at failure was 20 mm. Determine the unconfined compressive strength of the clay. | CO4 | 5M |