18CED31

Hall Ticket Number:

| IV/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| December, 2021 | Civil Engineering |  |  |
| Seventh Semester | PRE-STRESSED CONCRETE |  |  |
| Time: Three Hours |  | Maximum: 50 Marks |  |
| Answer Question No.1 compulsorily. |  |  | (10X1 = 10 Marks) |
| Answer ONE question from each unit. |  |  | (4X10=40 Marks) |

| 1. | a) | what is the principle of prestressed concrete? | CO1 |  |
| --- | --- | --- | --- | --- |
|  | b) | What is pressure line in PSC beam? | CO1 |  |
|  | c) | Sketch cable profiles for point load at center and Udl load for a simply supported PSC beam? | CO1 |  |
|  | d) | Define deflection. | CO2 |  |
|  | e) | What is wooble effect? | CO2 |  |
|  | f) | What do you mean by class-1 and class-2 prestressed concrete structures | CO2 |  |
|  | g) | Define kern points | CO2 |  |
|  | h) | Define transmission length | CO3 |  |
|  | i) | What are the modes of failure due to shear? | CO3 |  |
|  | j) | Spalling tension and bursting tension develops in __________ zone. | CO3 |  |
| Unit -I |  |  |  |  |
| 2. | a) | What is the difference between pre-tensioned and post-tensioned concrete. | CO1 | 5M |
|  | b) | Describe briefly the FREYSSINET system of prestressing with neat sketch | CO1 | 5M |
| (OR) |  |  |  |  |
| 3. | a) | What are the types of prestressing systems, explain? | CO1 | 5M |
|  | b) | Simply supported rectangular concrete beam 150mm wide by 300mm deep spanning over 8m. is presented by a straight cable carrying an effective prestressing force of 300kN located at an eccentricity of 50mm. The beam supports a live load of 2.6kN/m. Calculate the resultant stress distribution for the central cross section of the beam. Take density of concrete is 24kN/m3. | CO1 | 5M |
| Unit -II |  |  |  |  |
| 4. | a) | Explain facors influencing deflections of PSC menbers. | CO2 | 4M |
|  | b) | A concrete beam is prestressed by a cable carrying an initial prestressing force of 300 kN. The cross-sectional area of the wires in the cable is 300 mm2. Calculate the percentage loss of stress in the cable due to shrinkage of concrete using IS: 1343 recommendations assuming the beam to be, (i) pre-tensioned and (ii) post-tensioned. Assume Es = 210 kN/m2 and age of concrete at transfer = 8 days. | CO2 | 6M |
| (OR) |  |  |  |  |
| 5. |  | A prestressed concrete beam of rectangular section 300 mm wide and 600 mm deep has span of 10m. The effective pretstressing force is 980 kN at an eccentricity of 120 mm. the dead load of the beam is 450 kN/m and the beam has to carry a live load of 7.50 KN/m. determine the extreme stresses. | CO2 | 10M |
| Unit -III |  |  |  |  |
| 6. |  | A pretensioned prestressed concrete beam having a rectangular section, 150mm wide and 350 mm deep, has an effective cover of 50 mm. if fck = 45 N/sqmm, fp=1500 N/sqmm, and the area of prestressing steel is 500sqmm. Determine the ultimate flexure strength of the section using IS: 1343 code provisions. | CO3 | 10M |
| (OR) |  |  |  |  |
| 7. | a) | Outline the assumptions made in analysis of flexure. | CO3 | 4M |
|  | b) | A pretensioned prestressed concrete beam having a rectangular section, 200mm wide and 350 mm deep, has an effective cover of 50 mm. if fck= 45 N/sqmm, fp=1500 N/sqmm, and the area of prestressing steel is500sqmm. Determine the ultimate flexure strength of the section using IS: 1343 code provisions. | CO3 | 6M |
| Unit -IV |  |  |  |  |
| 8. |  | Find The support section of a prestressed concrete beam 100mm wide and 250mm deep is required to support an ultimate shear force of 80kN. The compressive prestress at the centroidal axis is 5N/mm2.The cover to the tension reinforcement is 50mm. If the characteristic tensile strength of the stirrups is 415N/mm2, design suitable shear reinforcement in the section using IS code recommendations. | CO4 | 10M |
| (OR) |  |  |  |  |
| 9. |  | Explain the end block/ anchorage zone stress analysis in post tensioned beam. | CO4 | 10M |

.