18CED61

Hall Ticket Number:

| IV/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| May, 2022 | Civil Engineering |  |  |
| Eight Semester | EARTHQUAKE RESISTANT DESIGN OF STRUCTURES |  |  |
| Time: Three Hours |  | Maximum: 50 Marks |  |
| Answer Question No.1 compulsorily. |  |  | (10X1 = 10 Marks) |
| Answer ONE question from each unit. |  |  | (4X10=40 Marks) |

| 1. | a) | Define: i) Time period ii) Critical damping |  |  |
| --- | --- | --- | --- | --- |
|  | b) | What are the causes of earthquakes? |  |  |
|  | c) | What are the major aspects involved in seismic-resistant building construction. |  |  |
|  | d) | Write short notes on seismograph. |  |  |
|  | e) | Explain the different factors responsible for the occurrence of earthquakes. |  |  |
|  | f) | Define liquefaction & what are the types of liquefaction |  |  |
|  | g) | Define base shear. |  |  |
|  | h) | What do you mean by soft storey. |  |  |
|  | i) | List out various load combinations as per IS code. |  |  |
|  | j) | Why beam column joints are so important in earthquake resistant design of structures. |  |  |
| Unit -I |  |  |  |  |
| 2. |  | Derive the equation of motion of a damped single degree freedom system subjected to free vibration. | 10M |  |
|  |  | (OR) |  |  |
| 3. | a) | Derive the equation of motion of a damped single degree freedom system subjected to earthquake excitation (base excitation). | 5M |  |
|  | b) | Discuss about Response spectrum of a structure | 5M |  |
|  |  | Unit -II |  |  |
| 4. | a) | Discuss briefly the two measures of an earthquake. | 5M |  |
|  | b) | Explain about seismic zoning of India as per IS 1893-Part1:2002? | 5M |  |
|  |  | (OR) |  |  |
| 5. | a) | Explain the method of Pseudo-static analysis for evaluating the inertial slope stability. |  | 5M |
|  | b) | List out the effects of liquefaction on built environment |  | 5M |
|  |  | Unit -III |  |  |
| 6. |  | Explain in detail about the calculation of lateral force due to earthquake using equivalent static method as per IS: 1893-2002 (part-I). | 10M |  |
|  |  | (OR) |  |  |
| 7. |  | A framed RCC building of single storey has dimensions of 5mX5mX3.2m high. Columns are located at 12M four corners of the building. The columns are 230 mm X300 mm in size, slab thickness is 120mm. the live load on the floor is 3kN/m2 and the building is on hard rock and falling in seismic zone IV. Analyze a plane frame of the building for dead load and live load combinations. | 10M |  |
|  |  | Unit -IV |  |  |
| 8. |  | Find Design a long column as per IS13920 for the following data obtained from the analysis. Axial load = 600kN; Moment = 300kNm; Shear force = 90kN. Assume the size of the column as 450 x 650mm. Use M20 grade concrete and Fe415 grade steel. Show that detd detailing of the column with a neat sketch. | 10M |  |
|  |  | (OR) |  |  |
| 9. | a) | Explain different house types and damages and also discuss different cause and location of damage. | 5M |  |
|  | b) | Define retrofitting. Explain retrofitting techniques. | 5M |  |