18EC504

Hall Ticket Number:

| III/IV B.Tech (Regular / Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2022 | Electronics & Communication Engineering |  |  |
| Fifth Semester | Digital Signal Processing |  |  |
| Time: Three Hours |  | Maximum: 50 Marks |  |
| Answer Question No.1 compulsorily. |  |  | (1X10 = 10 Marks) |
| Answer ONE question from each unit. |  |  | (4X10=40 Marks) |

| 1. | a) | What are the advantages of DSP over ASP? | CO1 |  |
| --- | --- | --- | --- | --- |
|  | b) | Give the relationship between impulse response and step response of LTI system. | CO1 |  |
|  | c) | Write any two properties related to ROC of Z-transform. | CO1 |  |
|  | d) | What are the applications of DFT? | CO2 |  |
|  | e) | Determine the number of complex multiplications required to find the 32-point DFT of a sequence using FFT algorithm. | CO2 |  |
|  | f) | What is aliasing-effect in impulse invariant transformation method? | CO3 |  |
|  | g) | What are the advantages of Chebyshev filters over Butterworth filters? | CO3 |  |
|  | h) | What is the advantage of Linear Phase realization of FIR systems? | CO4 |  |
|  | i) | What is Gibb’s phenomenon in FIR filter design using Rectangular Window? | CO4 |  |
|  | j) | What are the desirable characteristics of window function? | CO4 |  |
| Unit -I |  |  |  |  |
| 2. | a) | Determine whether the following system is Linear, Time-invariant, Causal and Stable with appropriate test conditions. y(n) = | CO1 | 5M |
|  | b) | Determine the impulse response and step response of the system described by the difference equation y(n) + y(n–1) – 2y(n–2) = x(n–1) + 2x(n–2). | CO1 | 5M |
| (OR) |  |  |  |  |
| 3. | a) | Consider an LTI system with system function H(z) = with ROC ¦z¦ > 1 - Sketch the pole-zero plot. Is the system stable? - Determine the impulse response of the system | CO1 | 5M |
|  | b) | Given an LTI system with impulse response h(n) = (0.2)n u(n). Find the response of the system to the input x(n) = (0.5)n u(n). | CO1 | 5M |
| Unit -II |  |  |  |  |
| 4. | a) | Compute the 4-point DFT of the sequence x(n) = (3,2,2,3) and also sketch its magnitude and phase spectrum. | CO2 | 5M |
|  | b) | Find the 8-point DFT of the following signal using radix-2 DIT FFT algorithm x(n) = (1, -2, 3, 2, -3, 2, 1, 3) | CO2 | 5M |
| (OR) |  |  |  |  |
| 5. | a) | State and prove the following properties of DFT - Circular Convolution ii) Periodicity property | CO2 | 5M |
|  | b) | Find the circular convolution of the following signals for N= 5. x1(n) = 3δ(n) + δ(n-1) - δ(n-2) - 2δ(n-4) x2(n) = δ(n) - 2δ(n-2) + 2δ(n-3) + 3(n-4) | CO2 | 5M |
| P.T.O. 18EC504 Unit -III |  |  |  |  |
| 6. | a) | For the analog transfer function ; Determine H(z) of the corresponding digital filter using impulse invariant method with T= 1 sec. | CO3 | 5M |
|  | b) | Realize the following system using Cascade and Parallel form structures. H(z) = | CO3 | 5M |
| (OR) |  |  |  |  |
| 7. | a) | Derive the relationship between s-plane poles and z-plane poles in impulse invariant method. | CO3 | 5M |
|  | b) | Design a digital Butterworth filter that satisfies the following constraints using bilinear transformation method. Assume T= 1 sec. 0.707 ≤ H(ejω) ≤ 1 ; 0 ≤ ω ≤ π/2 H(ejω) ≤ 0.2; 3π/4 ≤ ω ≤ π | CO3 | 5M |
| Unit -IV |  |  |  |  |
| 8. | a) | Derive the expression for frequency response of FIR filter when the impulse response is symmetric & N is odd. | CO4 | 5M |
|  | b) | Design a FIR LPF with a cutoff frequency of 2 K Hz and sampling rate of 5 KHz with 11 samples using Hamming window. Also sketch the impulse response of the filter. | CO4 | 5M |
| (OR) |  |  |  |  |
| 9. | a) | Compare various windows used in windowing technique. | CO4 | 5M |
|  | b) | Design an ideal High pass filter with the desired response H(ejω) = e-j3ω ; π/4 ≤ ¦ω¦ ≤ π = 0 ; -π/4 ≤ ω ≤ π/4 Find h(n) for N =7 using Hamming window. | CO4 | 5M |