| 20EE402 Hall Ticket Number: II/IV B.Tech (Regular) DEGREE EXAMINATION. August,2022; Electrical and Electronics Engineering. Fourth Semester; Digital Electronics. Time: Three Hours; Maximum:70 Marks. |  |
| --- | --- |
| Answer question 1 compulsory.; (14X1 = 14 Marks). Answer one question from each unit.; (4X14=56 Marks). |  |
|  | 1. | a) | What is an acceptable low level voltage for a TTL IC?. |  |  |  |  |
|  |  | b) | Find the binary equivalent of (374.26)8. |  |  |  |  |
|  |  | c) | Obtain the truth table for EX-NOR gate. |  |  |  |  |
|  |  | d) | What is the difference between Latch and Flip-flop? |  |  |  |  |
|  |  | e) | What are the applications of Octal number system? |  |  |  |  |
|  |  | f) | What are the Limitations of Karnaugh maps? |  |  |  |  |
|  |  | g) | Define Pair, Quad, and Octet? |  |  |  |  |
|  |  | h) | Define Fan-in and Fan-out |  |  |  |  |
|  |  | i) | What is Half-adder? |  |  |  |  |
|  |  | j) | What is De-Multiplexer? |  |  |  |  |
|  |  | k) | Write any differences between Combinational Circuits and Sequential Circuits? |  |  |  |  |
|  |  | l) | Define Rise-time? |  |  |  |  |
|  |  | m) | List the different types PLDs. |  |  |  |  |
|  |  | n) | What is the difference between PALs and PLAs? |  |  |  |  |
|  | Unit –I |  |  |  |  |  |  |
|  | 2. | a) | Perform the subtraction (11010- 1101) with the following binary numbers using (i) 2’s complement and (ii) l’s complement. Check the answer by straight subtraction. | CO1 | 7M |  |  |
|  |  | b) | Construct the hamming code for the data bits 1101 | CO1 | 7M |  |  |
|  | (OR) |  |  |  |  |  |  |
|  | 3. | a) | Convert the following into canonical SOP (i) AB + B′C + A′B′C′ (ii) BC + AC′ + AB | CO1 | 7M |  |  |
|  |  | b) | Briefly explain about TTL logic gate? | CO1 | 7M |  |  |
|  | Unit –II |  |  |  |  |  |  |
|  | 4. | a) | Simplify f(w, x, y, z) = ∑(0, 1, 2, 8, 12, 13, 14) + d(3, 5, 10, 15) with K-Map. | CO2 | 7M |  |  |
|  |  | b) | Write short notes on Parity generator and checker | CO2 | 7M |  |  |
|  | (OR) |  |  |  |  |  |  |
|  | 5. | a) | Simplify the following Boolean function by using the tabulation method: F = ∑(0, 1, 2, 8, 10, 11, 14, 15) | CO2 | 7M |  |  |
|  |  | b) | Implement a combinational circuit which convert four bit excess 3 code to four bit BCD code. | CO2 | 7M |  |  |
|  | Unit –III |  |  |  |  |  |  |
|  | 6. | a) | Covert T flip-flip to D flip-flop. | CO3 | 7M |  |  |
|  |  | b) | Draw the diagram of a 3-bit binary Asynchronous down-counter . | CO3 | 7M |  |  |
|  | (OR) |  |  |  |  |  |  |
|  | 7. | a) | Briefly explain master slave JK Flip-flop. | CO3 | 7M |  |  |
|  |  | b) | Design a MOD 10 synchronous binary up counter using T flip-flop and other necessary logic gates. | CO3 | 7M |  |  |
|  | Unit –IV |  |  |  |  |  |  |
|  | 8. | a) | Explain R-2R ladder D/A converter. | CO4 | 7M |  |  |
|  |  | b) | Describe the working of a successive approximation A/D converter with the help of a suitable diagram | CO4 | 7M |  |  |
|  | (OR) |  |  |  |  |  |  |
|  | 9. | a) | How does the PLA differ from ROM from the point of view of functionality | CO4 | 7M |  |  |
|  |  | b) | Describe the differences between the PLA and PAL. | CO4 | 7M |  |  |