18EI503

Hall Ticket Number:

| III/IV B.Tech(Regular / Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2022 | Electronics and Instrumentation Engineering |  |  |
| Fifth Semester | Linear Integrated Circuits and Applications |  |  |
| Time: Three Hours |  | Maximum: 50 Marks |  |
| Answer Question No.1 compulsorily. |  |  | (10X1 = 10 Marks) |
| Answer ONE question from each unit. |  |  | (4X10=40 Marks) |

| 1. | a) | Why open-loop op-amp configurations are not used in linear applications?Justify | CO1 |  |
| --- | --- | --- | --- | --- |
|  | b) | Calculate the CMRR of given opamp having differential mode gain Ad=1000. commonmode gain Ac =400. | CO1 |  |
|  | c) | Draw the voltage transfer curve of an op-amp? | CO1 |  |
|  | d) | List the applications of comparator. | CO2 |  |
|  | e) | What is Barkhausen criterion? | CO2 |  |
|  | f) | Draw the circuit diagram of negative clipper. | CO3 |  |
|  | g) | What is the draw back of binary weighted resistor type D/A convertor? | CO3 |  |
|  | h) | Determine the quality factor of a band-pass filter with a bandwidth of 250Hz and center frequency of 866Hz. | CO4 |  |
|  | i) | Why lock in range of PLL is greater than capture range? | CO4 |  |
|  | j) | List any 4 applications of 555 Astable multivibrator. | CO4 |  |
| Unit -I |  |  |  |  |
| 2. | a) | Draw the block diagram of an op-amp and briefly explain the function of each block, also outline the ideal characteristics of op-amp. | CO1 | 5M |
|  | b) | What are the important features of a 3 op-amps instrumentation amplifier and derive the expression for output voltage. | CO2 | 5M |
| (OR) |  |  |  |  |
| 3. | a) | What is the limitation of ideal integrator? Explain how do you overcome the limitation using practical integrator? | CO2 | 5M |
|  | b) | Explain the basic principle of operation of precision Full wave rectifier with a neat sketch and necessary equations? | CO2 | 5M |
| Unit -II |  |  |  |  |
| 4. | a) | Explain the operation of voltage controlled oscillator with neat functional diagram and derive the expression for free running frequency. | CO2 | 5M |
|  | b) | Explain the basic principle of operation of Ideal comparator. What are the limitations of Op-amp as a comparator. | CO2 | 5M |
| (OR) |  |  |  |  |
| 5. | a) | With a neat circuit diagram, necessary waveforms and relevant equations, explain square wave generator using op-amp. | CO2 | 5M |
|  | b) | Design a Triangular wave generator so that f o =1KHz and Vo(P-P) =7V with ±V sat =11V. Assume R 2 =1KΩ, C1=0.01µF. | CO2 | 5M |
| Unit -III |  |  |  |  |
| 6. | a) | Explain the operation of a sample and hold circuit using op-amp with a neat sketch and give its applications. | CO3 | 5M |
|  | b) | Explain the operation of successive approximation type A-D converter with an example. | CO3 | 5M |
| (OR) |  |  |  |  |
| 7. | a) | Explain the basic principle of operation of positive & negative clampers with neat wave forms. | CO3 | 5M |
|  | b) | Explain the construction and working of R-2R Ladder type D/A converter with a neat circuit diagram and example. | CO3 | 5M |
| Unit -IV |  |  |  |  |
| 8. | a) | Explain the application of 555 timer in monostable mode as - Frequency divider - Missing pulse detector | CO4 | 5M |
|  | b) | Design a first order high pass filter at a cut off frequency of 15.9KHz,with a pass band of 1.5 and sketch its frequency response(Assume C=0.01µF). | CO4 | 5M |
| (OR) |  |  |  |  |
| 9. | a) | Describe the concept of PLL with neat block diagram | CO4 | 4M |
|  | b) | Derive the general transfer function of second order active low pass filter. | CO4 | 6M |