18EI504

Hall Ticket Number:

| III/IV B.Tech (Regular / Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2022 | Electronics and Instrumentation Engineering |  |  |
| Fifth Semester | Microcontrollers |  |  |
| Time: Three Hours |  | Maximum: 50 Marks |  |
| Answer Question No.1 compulsorily. |  |  | (1X10 = 10 Marks) |
| Answer ONE question from each unit. |  |  | (4X10=40 Marks) |

| 1. | a) | What is a debugger | CO1 |  |
| --- | --- | --- | --- | --- |
|  | b) | What is the default memory location for stack pointer | CO1 |  |
|  | c) | What for EA pin is used in 8051 | CO1 |  |
|  | d) | What is time delay. How it can be calculated. | CO2 |  |
|  | e) | Explain EQU assembler directive | CO2 |  |
|  | f) | How many I/O ports are available in 8255 | CO2 |  |
|  | g) | Write the differences between common anode and common cathode seven segment display | CO2 |  |
|  | h) | What is the program memory | CO3 |  |
|  | i) | Explain RISC architecture | CO3 |  |
|  | j) | What is MSSP module. List the advantages of it. | CO3 |  |
| Unit -I |  |  |  |  |
| 2. | a) | Compare Microprocessors and Microcontrollers | CO1 | 5M |
|  | b) | Explain the pin configuration of 8051 in detail | CO1 | 5M |
| (OR) |  |  |  |  |
| 3. | a) | Explain about various addressing modes of 8051 | CO1 | 5M |
|  | b) | Explain about TCON and SCON registers of 8051 | CO1 | 5M |
| Unit -II |  |  |  |  |
| 4. | a) | Explain about conditional branch instructions of 8051 | CO2 | 5M |
|  | b) | Write an ALP to arrange the numbers in ascending order | CO2 | 5M |
| (OR) |  |  |  |  |
| 5. | a) | Explain about logical instructions of 8051 | CO2 | 5M |
|  | b) | Write an ALP to generate the Fibonacci series. | CO2 | 5M |
| Unit -III |  |  |  |  |
| 6. | a) | Explain how a temperature sensor can be interfaced to 8051 | CO3 | 5M |
|  | b) | Write an ALP to interface a seven segment display to 8051 and display 0 to 9 | CO3 | 5M |
| (OR) |  |  |  |  |
| 7. |  | With the help of neat sketch and program explain how a keyboard can be interfaced. | CO3 | 10M |
| Unit -IV |  |  |  |  |
| 8. | a) | Explain various Port pins of 16F877 and their alternate functions. | CO4 | 5M |
|  | b) | Explain about memory organization in PIC 16F877 | CO4 | 5M |
| (OR) |  |  |  |  |
| 9. | a) | Write about INTCON (Interrupt Controlling register) | CO4 | 5M |
|  | b) | Explain about Timers used in PIC 16F877. | CO4 | 5M |