18EID61

Hall Ticket Number:

| IV/IV B. Tech (Regular) DEGREE EXAMINATION |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| May,2022 |  |  | Electronics & Instrumentation Engineering |  |  |  |  |  |
| Eighth Semester |  |  | Optoelectronics &Laser Instrumentation |  |  |  |  |  |
| Time: Three Hours |  |  |  | Maximum : 50 Marks |  |  |  |  |
| Answer Question No.1 compulsorily. |  |  |  |  | (1X10 = 10 Marks) |  |  |  |
| Answer ONE question from each unit. |  |  |  |  | (4X10=40 Marks) |  |  |  |
| 1 | Answer all questions |  |  |  |  | (1X10=10 Marks) |  |  |
|  | a) | What is meant by critical angle? |  |  |  |  |  |  |
|  | b) | Distinguish between meridional rays and skew-rays |  |  |  |  |  |  |
|  | c) | Distinguish between inter and intra model dispersion? |  |  |  |  |  |  |
|  | d) | Define mode locking of lasers |  |  |  |  |  |  |
|  | e) | Distinguish between spontaneous and stimulated emissions |  |  |  |  |  |  |
|  | f) | Name two LED Structures used in intruder alarms and in TV remotes. |  |  |  |  |  |  |
|  | g) | Define internal quantum efficiency of LED. |  |  |  |  |  |  |
|  | h) | List various types of optical detectors used in fiber optic communication system? |  |  |  |  |  |  |
|  | i) | Mention the advantages of fiber optic sensors |  |  |  |  |  |  |
|  | j) | Define the quantum efficiency of an Avalanche photo diode. |  |  |  |  |  |  |
| UNIT I |  |  |  |  |  |  |  |  |
| 2 | a) | What are the functions of the core and cladding in an optical fiber? Why should their refractive indices be different? Would it be possible for the light to be guided without cladding? |  |  |  |  | 6M |  |
|  | b) | A multimode step index fiber with a core diameter of 80µm & a relative refractive index difference of 1.5% is operating at a wavelength of 0.85µm. If the core refractive index is 1.48, estimate (a) the normalized frequency for the fiber (b) the number of guided modes. |  |  |  |  | 4M |  |
| (OR) |  |  |  |  |  |  |  |  |
| 3 |  | Classify basic attenuation mechanisms in an optical fiber? Discuss them in detail. |  |  |  |  | 10M |  |
| UNIT II |  |  |  |  |  |  |  |  |
| 4 | a) | Explain the principle and operation of laser Doppler velocity meter. |  |  |  |  | 5M |  |
|  | b) | Compare the features of gain guided and index guided lasers |  |  |  |  | 5M |  |
| (OR) |  |  |  |  |  |  |  |  |
| 5 | a) | Briefly discuss the role of laser in material processing |  |  |  |  | 5M |  |
|  | b) | Explain any one medical application of laser with neat diagram |  |  |  |  | 5M |  |
| UNIT III |  |  |  |  |  |  |  |  |
| 6 | a) | Illustrate the operation of fabry perot interferometric fiber optic sensor used for pressure measurement |  |  |  |  | 5M |  |
|  | b) | Describe the operation of Mach-Zehnder interferometric fiber optic sensor used for force measurements |  |  |  |  | 5M |  |
| (OR) |  |  |  |  |  |  |  |  |
| 7 | a) | Outline the construction and operation of super luminescent LED |  |  |  |  | 6M |  |
|  | b) | Explain the characteristics of LED |  |  |  |  | 4M |  |
| UNIT IV |  |  |  |  |  |  |  |  |
| 8 | a) | Illustrate the principle and operation of reach through Avalanche photo diode (RAPD). |  |  |  |  | 5M |  |
|  | b) | A P-n photodiode has a quantum efficiency of 70% for photons of energy 1.52*10-19J. Calculate (a) wavelength at which the diode is operating & (b) the optical power required to achieve a photocurrent of 3µA when the wavelength of incident photons is that calculated in part (a) |  |  |  |  | 5M |  |
| (OR) |  |  |  |  |  |  |  |  |
| 9 | a) | With a neat diagram discuss the operation of Acoustic optic modulator. |  |  |  |  | 5M |  |
|  | b) | Explain any one type of polarization maintaining fiber with neat diagram |  |  |  |  | 5M |  |