18EII04

Hall Ticket Number:

| IV/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| May, 2022 | Common to all branches |  |  |
| Eighth Semester | Sensors and Signal Conditioning |  |  |
| Time: Three Hours |  | Maximum: 50 Marks |  |
| Answer Question No.1 compulsorily. |  |  | (10X1 = 10 Marks) |
| Answer ONE question from each unit. |  |  | (4X10=40 Marks) |

| 1. | a) | Define hysteresis. |  |
| --- | --- | --- | --- |
|  | b) | What is an active transducer? |  |
|  | c) | Specify the significance of signal conditioning circuit. |  |
|  | d) | What is the working principle of LVDT? |  |
|  | e) | What is meant by cold junction compensation in thermocouple? |  |
|  | f) | What are the applications of electrochemical sensor? |  |
|  | g) | What is piezo electric effect? |  |
|  | h) | Classify digital encoders. |  |
|  | i) | What are the sources of Noise in electronic circuits. |  |
|  | j) | How does a resonant sensor work? |  |
| Unit -I |  |  |  |
| 2. | a) | Describe the working of any two primary pressure sensors. | 5M |
|  | b) | A temperature sensor has a range of 20oC to 250oC. If the measured temperature is 55o C. Calculate the error if the accuracy is: (a) ± 0.5% Full scale deflection (b) ± 0.75% of span. What is the possible temperature in each case? | 5M |
|  |  | (OR) |  |
| 3. | a) | A strain gauge having an unstrained resistance of 120 Ω and a gauge factor of 2.1 is bonded onto a steel girder so that it experiences a tensile stress of 108 Pa. If Young’s modulus for steel is 2 × 1011 Pa, calculate the strained resistance of the gauge. | 5M |
|  | b) | Discuss signal conditioning for a displacement measuring potentiometer. | 5M |
|  |  | Unit -II |  |
| 4. | a) | Explain how displacement is measured using Capacitive transducers. | 5M |
|  | b) | The output of a LVDT is connected to a 4 V voltmeter through an amplifier whose amplification factor is 500. An output of 1.8mV appears across the terminals of LVDT when the core moves through a distance of 0.6mm. If the milli-voltmeter scale has 100 divisions and the scale can be read to 1⁄4 of a division, calculate: (i) the sensitivity of LVDT (ii) The resolution of the instrument in mm. | 5M |
|  |  | (OR) |  |
| 5. | a) | Explain the working principle of electromagnetic flow meter with a neat sketch. | 5M |
|  | b) | Discuss the signal conditioning for a differential capacitive sensor using ac bridge to double sensitivity. | 5M |
|  |  | Unit -III |  |
| 6. | a) | Summarize the laws for temperature measurement using thermocouples. | 5M |
|  | b) | Draw the circuit of charge amplifier and explain the need of it. | 5M |
|  |  | (OR) |  |
| 7. | a) | Explain the working principle of potentiometric oxygen sensor with a neat sketch. | 5M |
|  | b) | Discuss the principle of operation for a chopper amplifier with a neat sketch. | 5M |
|  |  | Unit -IV |  |
| 8. | a) | Find Draw the diagram of incremental position encoder and explain its operation | 5M |
|  | b) | Draw the block diagram of quartz digital thermometer and explain its operation. | 5M |
|  |  | (OR) |  |
| 9. | a) | Draw the block diagram of Frequency meter and explain its operation. | 5M |
|  | b) | With a neat block diagram, explain various components of an intelligent sensor. | 5M |