| 20ME403 Hall Ticket Number: II/IV B.Tech (Regular) DEGREE EXAMINATION. August 2022; Mechanical Engineering. Fourth Semester; Kinematics of Machines. Time: Three Hours; Maximum:70 Marks. |  |
| --- | --- |
| Answer question 1 compulsory.; (14X1 = 14 Marks). Answer one question from each unit.; (4X14=56 Marks). |  |

| 1. | a) | Can a machine be called as a mechanism? | CO1 |  |
| --- | --- | --- | --- | --- |
|  | b) | How many degrees of freedom contained by a four bar mechanism? | CO1 |  |
|  | c) | Give an example for completely constrained motion. | CO1 |  |
|  | d) | Differentiate between higher pair and lower pair? | CO2 |  |
|  | e) | What are the components of linear acceleration of a kinematic link? | CO2 |  |
|  | f) | What is significance of Coriolis component of acceleration? | CO2 |  |
|  | g) | Find the resultant acceleration of 100mm diameter crank rotating at a constant angular velocity of 5m/s, at the crank pin position. | CO3 |  |
|  | h) | What are various types of instantaneous centers of rotation? | CO3 |  |
|  | i) | State the three centers in a line theorem. | CO3 |  |
|  | j) | Why a roller follower is preferred to that of a knife-edged follower in cam mechanisms? | CO3 |  |
|  | k) | What is interference in Involute gears? | CO4 |  |
|  | l) | Mention any one advantage of cycloidal gears. | CO4 |  |
|  | m) | Define Pressure angle regarding gears. | CO4 |  |
|  | n) | Define ‘pitch circle’ for a spur gear. | CO4 |  |
| Unit –I |  |  |  |  |
| 2. | a) | Classify the Kinematic pairs. | CO1 | 7M |
|  | b) | Sketch any two inversions of double slider crank mechanism. | CO1 | 7M |
| (OR) |  |  |  |  |
| 3. | a) | Explain the kutz-bach criteria for planar mechanisms with two examples. | CO1 | 7M |
|  | b) | Sketch and explain the applications of any one quick return mechanism. | CO1 | 7M |
| Unit –II |  |  |  |  |
| 4. | a) | Find the angular acceleration of the link CD for the mechanism shown. | CO2 | 7M |
|  | b) | Explain the concept of velocity of rubbing in the quadratic mechanisms. | CO2 | 7M |
| (OR) |  |  |  |  |
| 5. | a) | In a fourlink mechanism, the dimensions of the links are as follows; AB=50 mm, BC=66 mm, CD=56 mm, and AD=100 mm. At the instant Angle DAB=600, the link AB has an angular velocity of 10.5 rad/sec, in the counter clockwise direction. Determine - Velocity of point C - Velocity of the point E on the link BC when BE=40mm, - Angular velocity of the links BC and CD | CO2 | 14M |
| P.T.O 20ME403 |  |  |  |  |
| Unit –III |  |  |  |  |
| 6. | a) | Draw and locate all the possible instantaneous centers of rotation for a single slider crank mechanism. | CO3 | 7M |
|  | b) | Find the number of instantaneous centers of rotation for the mechanism. Also locate all I-centers choosing any dimensions. | CO3 | 7M |
| (OR) |  |  |  |  |
| 7. | a) | A cam drives a flat reciprocating follower in the following manner: During first 1200 rotation of the cam, follower moves outwards through a distance of 20mm with SHM. The follower dwells during next 300 of cam rotation. During next 1200 of cam rotation, the follower moves inwards with SHM. The follower dwells for the next 900 of cam rotation. Draw the displacement diagram to a suitable scale. Lift of the cam is 50mm. | CO3 | 7M |
|  | b) | Construct the profile of a cam to suit the following specifications: Cam shaft diameter = 40 mm ; Least radius of cam= 25 mm; Angle of lift = 1200; Angle of fall = 1500 ; Lift of the follower = 40 mm ; Number of pauses are two of equal interval between motions. During the lift and fall, the motion is uniform velocity. The speed of the cam shaft is uniform. The line of stroke of the follower is in line with center of the cam. | CO3 | 7M |
| Unit –IV |  |  |  |  |
| 8. | a) | Determine the number of pairs of teeth in contact at any given instant, if two involute gears of 18 teeth of pressure angle 19.5 degrees have addendum of 0.8 module. | CO4 | 7M |
|  | b) | Explain the phenomena of interference in the gears. | CO4 | 7M |
| (OR) |  |  |  |  |
| 9. | a) | An epicyclic gear is shown below. The Gears A, D and E are free to rotate on the axis P. The compound gear B and C rotate together on the axis Q at the end of arm F. All the gears have a equal pitch. The number of external teeth on the gears A, B and C are 18, 45, and 21 respectively. The gears D and E are annular gears. The gear A rotates at 100 rpm in CCW direction and gear rotates at 450 rpm CW. Find the speed and direction of the arm and gear E. Assume any data required. | CO4 | 14M |