20CE303

Hall Ticket Number:

| II/IV B.Tech (Regular / Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| February, 2023 | Solid Mechanics |  |  |
| Third Semester | Civil Engineering |  |  |
| Time: Three Hours |  | Maximum:70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

| 1. | a) |  |  | Sketch the stress–strain diagram for mild steel | CO1 | L3 |  | 1M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | b) |  |  | Define Poisson’s ratio | CO1 | L1 |  | 1M |
|  | c) |  |  | What are the Assumptions followed in thin pressure vessels? | CO1 | L1 |  | 1M |
|  | d) |  |  | What is the relation between rate of loading, SF and BM? | CO2 | L1 |  | 1M |
|  | e) |  |  | Explain point of contra flexure | CO2 | L2 |  | 1M |
|  | f) |  |  | Determine the section modulus of rectangular cross section b x d | CO2 | L1 |  | 1M |
|  | g) |  |  | State assumptions made in theory of pure bending | CO3 | L2 |  | 1M |
|  | h) |  |  | Draw the shear stress distribution for ‘ T’ | CO3 | L1 |  | 1M |
|  | i) |  |  | Define torsion . | CO4 | L1 |  | 1M |
|  | j) |  |  | What is stiffness of the spring? | CO4 | L1 |  | 1M |
|  | k) |  |  | Define section modulus and write the section modulus for circular section | CO4 | L1 |  | 1M |
|  | l) |  |  | A solid circular shaft is subjected to a torque of 20KNm.If the maximum permissible shear stress is 50N/mm2, find the suitable minimum diameter for the shaft. | CO4 | L1 |  | 1M |
|  | m) |  |  | Define torsional Rigidity | CO4 | L1 |  | 1M |
|  | n) |  |  | Define spring constant | CO4 | L1 |  | 1M |
|  |  | Unit-I |  |  |  |  |  |  |
| 2. |  |  | Derive the relation between the three elastic constants E,G and K. |  | CO1 | L3 | 14M |  |
|  |  |  | (OR) |  |  |  |  |  |
| 3. | a) |  | A cylindrical vessel whose ends are closed by means of rigid flange plates is made of steel plate 3mm thick. The internal length and diameter of vessel are 50cm and 25cm respectively. Determine the longitudinal and circumferential stresses in the cylindrical shell due to an internal fluid pressure of 3MN/m2 . Also, calculate increase in length, diameter and volume of the vessel. Take: Young’s modulus = 200GN/m2 and Poisson’s ratio = 0.3 |  | CO1 | L4 | 8M |  |
|  | b) |  | Write any two differences between cylindrical shell and spherical shell |  | CO1 | L1 | 6M |  |
|  |  |  | Unit-II |  |  |  |  |  |
| 4. | a) |  | Define shear force and bending moment |  | CO2 | L2 | 4M |  |
|  | b) |  | A simply supported beam of span 10 m carry UDL of 10kN/m over its entire length of 10 m and also a point load of 50KN at its mid-span . Draw SF and BM diagram. |  | CO2 | L2 | 10M |  |
|  |  |  | (OR) |  |  |  |  |  |
| 5. | a) |  | Draw the shear force and bending moment diagrams for the SSB loaded as shown in fig. |  | CO2 | L3 | 10M |  |
|  | b) |  | Determine the maximum shear force and bending moment of a simply supported beam carrying a uniformly distributed load of w per unit run over the whole span. |  | CO2 | L1 | 4M |  |
|  |  |  | Unit-III |  |  |  |  |  |
| 6. |  |  | A rolled steel joint of symmetrical I-section has flange width of 300 mm and 20 mm thick with a web thickness of 20 mm and overall depth of I-section is 600 mm. If this beam carries a UDL of 40kN/m over the simply supported beam of span 10 m, find the maximum shear stress produced in the beam. |  | CO3 | L4 | 14M |  |
| (OR) |  |  |  |  |  |  |  |  |
| 7. | a) |  | Derive Pure Bending Equation |  | CO3 | L3 | 10M |  |
|  | b) |  | Determine the shear stress at the level of neutral axis, if a beam has a triangle cross section having base “b” and altitude “h”. Let the shear force be subjected is F. |  | CO3 | L2 | 4M |  |
|  |  |  | Unit-IV |  |  |  |  |  |
| 8. | a) |  | What are the assumptions made in torsion equation? |  | CO4 | L1 | 4M |  |
|  | b) |  | Derive Pure Torsional Equation. |  | CO4 | L2 | 10M |  |
|  |  |  | (OR) |  |  |  |  |  |
| 9. | a) |  | When the helical extension spring is subjected to axial tensile force, the type of stress induced in the spring wire is? |  | CO4 | L3 | 4M |  |
|  | b) |  | A close coiled helical spring has 10 turns wire of diameter 15mm ,the mean radius of the coil is 100mm,find the maximum stress and elongation of the spring under an axial load of 1KN (G=85GPa) |  | CO4 | L4 | 10M |  |