20CE403

Hall Ticket Number:

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August, 2023 | Civil Engineering |  |  |
| Fourth Semester | Mechanics of Materials |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | Define buckling load. | CO2 | L1 | 1M |
|  | b) | Define strain energy. | CO4 | L1 | 1M |
|  | c) | What is the radius of mohr’s circle for the stresses σx = 40 N/mm2, σy = 40 N/mm2,τxy = 40 N/mm2? | CO1 | L1 | 1M |
|  | d) | What is maximum strain energy theory? | CO2 | L1 | 1M |
|  | e) | Draw the kern of a rectangular section? | CO1 | L1 | 1M |
|  | f) | What is the equivalent length of a column has both ends are fixed? | CO2 | L1 | 1M |
|  | g) | What is the slope and deflection at the free end of a cantilever has point load at its free end? | CO3 | L1 | 1M |
|  | h) | Define virtual work. | CO4 | L1 | 1M |
|  | i) | What is the deflection at center of a simply supported beam subjected to uniformly distributed load throughout its length? | CO3 | L1 | 1M |
|  | j) | What is the strain energy due to bending? | CO4 | L1 | 1M |
|  | k) | Write the Euler’s buckling load for long columns. | CO2 | L1 | 1M |
|  | l) | Define principle stresses. | CO1 | L1 | 1M |
|  | m) | Write a Mohr’s theorem I. | CO3 | L1 | 1M |
|  | n) | Define short column. | CO2 | L1 | 1M |
| Unit-I |  |  |  |  |  |
| 2 |  | At a point in an elastic material under strain, there are normal stresses of 50N/mm2 and 30N/mm2 respectively at right angles to each other with a shearing stress of 25 N/mm2. Find the principle planes and principle stresses. Also calculate maximum shear stress and its planes. | CO1 | L1 | 14M |
|  |  | (OR) |  |  |  |
| 3 | a) | Derive the expression for kern of a solid circular and rectangular section. | CO1 | L2 | 7M |
|  | b) | A short hollow circular column external diameter 400mm and internal diameter 350mm carries a load of 500kN through a bracket with an eccentricity 200mm. Find the maximum and minimum stresses due to this load. | CO1 | L1 | 7M |
| Unit-II |  |  |  |  |  |
| 4 |  | Explain assumption in Euler’s theory. Derive Euler’s buckling load for a column with one end fixed and other end hinged. | CO2 | L2 | 14M |
| (OR) |  |  |  |  |  |
| 5 | a) | Explain maximum strain energy theory | CO2 | L2 | 7M |
|  | b) | A steel bolt is subjected to a tensile load of 20kN along with a shear force of 10kN, the yield point for the material is 300N/mm2 and Poisson’s ratio is 0.30. Calculate the safe diameter according to maximum shear stress theory. | CO2 | L2 | 7M |
| Unit-III |  |  |  |  |  |
| 6 |  | Derive expression for slope and deflection of a cantilever beam of length L carrying a concentrated load W at free end. Use Double Integration method. | CO3 | L2 | 14M |
| (OR) |  |  |  |  |  |
| 7 |  | Find the slope and deflection at the end of the cantilever beam shown in fig. by moment of area method. EI = 4000kN-m2. | CO3 | L1 | 14M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Derive the expression for strain energy stored due to bending. | CO4 | L2 | 7M |
|  | b) | A cantilever beam carrying uniformly distributed load of intensity 30 N/m over its entire span of length 5m. Find the maximum deflection and slope produced by Strain energy method. | CO4 | L1 | 7M |
| (OR) |  |  |  |  |  |
| 9 |  | Find out horizontal and vertical deflection at C as shown in figure below. | CO4 | L2 | 14M |