20CE404

Hall Ticket Number:

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August, 2023 | Civil Engineering |  |  |
| Fourth Semester | Hydraulics & Hydraulic Machines |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

| 1 | a) | Define sub-critical, critical and super critical flow | CO1 |  | L1 | 1 |
| --- | --- | --- | --- | --- | --- | --- |
|  | b) | Differentiate between pipe flow and open channel flow | CO1 |  | L2 | 1 |
|  | c) | List different types of channel flows | CO1 |  | L1 | 1 |
|  | d) | List the types of hydraulic jumps | CO2 |  | L1 | 1 |
|  | e) | List classification of surface profiles | CO2 |  | L1 | 1 |
|  | f) | Give the equation for gradually varied flow in rectangular channels | CO2 |  | L2 | 1 |
|  | g) | Differentiate between inward and outward radial flow turbine | CO3 |  | L2 | 1 |
|  | h) | A jet of water strikes with a velocity of 40 m/s a flat plate inclined at 30° with the axis of the jet. If the cross-sectional area of the jet is 25 cm2. Determine the force exerted by the jet on the plate. | CO3 |  | L2 | 1 |
|  | i) | Define Draft tube | CO3 |  | L1 | 1 |
|  | j) | Draw the inlet and outlet velocity triangles for a centrifugal pump | CO4 |  | L2 | 1 |
|  | k) | Explain the statement of Buckingham pi theorem | CO4 |  | L2 | 1 |
|  | l) | Differentiate between section pipe and delivery pipe of the centrifugal pump. | CO4 |  | L1 | 1 |
|  | m) | Distinguish between a pump and turbine | CO3,4 |  | L2 | 1 |
|  | n) | Classify different types of turbines according to discharge | CO3 |  | L2 | 1 |
| Unit-I |  |  |  |  |  |  |
| 2 | a) | Determine the economical cross-section for an open channel of trapezoidal section with side slopes of 1 vertical to 2 horizontal, to carry 10 m3/s, the bed slope being 1/2000. Assume Manning coefficient as 0.022 |  | CO1 | L3 | 7M |
|  | b) | Derive the conditions for most economical trapezoidal channel section. |  | CO1 | L3 | 7M |
|  |  | (OR) |  |  |  |  |
| 3 | a) | Derive Chezy’s equation for discharge through open channel flow. |  | CO1 | L3 | 7M |
|  | b) | The discharge of water through a rectangular channel of width 8m, is 15 m3 /s when the depth of flow of water is 1.2 m. Calculate (i) Specific Energy (ii) Critical depth and critical velocity (iii) Value of minimum specific energy. |  | CO1 | L3 | 7M |
| Unit-II |  |  |  |  |  |  |
| 4 | a) | Explain the classification of channel slopes |  | CO2 | L3 | 7M |
|  | b) | Distinguish between GVF and RVF. |  | CO2 | L2 | 7M |
| (OR) |  |  |  |  |  |  |
| 5 | a) | A sluice gate discharges water into a horizontal rectangular channel with a velocity of 6m/s and the depth of flow is 0.4 m. The width of channel is 8m. Determine whether hydraulic jump will occur and if so, find its height and loss of head. Also determine the power loss in the hydraulic jump. |  | CO2 | L3 | 7M |
|  | b) | Obtain an expression for the depth after the hydraulic jump and the loss of head Due to the jump. Write the assumptions made |  | CO2 | L2 | 7M |
| Unit-III |  |  |  |  |  |  |
| 6 | a) | Derive the formula for force exerted by a jet of water when it strikes a fixed curved plate. |  | CO3 | L3 | 7M |
|  | b) | Describe the working of Pelton wheel along with its components with a neat sketch. |  | CO3 | L2 | 7M |
| (OR) |  |  |  |  |  |  |
| 7 | a) | With the help of neat diagrams explain the construction and working of aKaplan Turbine. |  | CO3 | L2 | 7M |
|  | b) | A jet of water 75 mm diameter strikes a curved plate at its centre with a velocity of 20 m/s. The curved plate is moving with a velocity of 8 m/s in the direction of the jet, the jet is deflected through an angle of 1650. Assuming the plate to be smooth, find i) Force exerted on the plate in the direction of jet ii) Power of the jet |  | CO3 | L3 | 7M |
| Unit-IV |  |  |  |  |  |  |
| 8 | a) | Explain Rayleigh’s Method of dimensional analysis with an example |  | CO4 | L3 | 7M |
|  | b) | Explain the different types of hydraulic similarities that must be exit between a prototype and model. |  | CO4 | L2 | 7M |
| (OR) |  |  |  |  |  |  |
| 9 | a) | Water is flowing through a pipe of diameter 30 cm at a velocity of 4 m/s. Find the velocity of oil flowing in another pipe of diameter 10 cm, if the condition of dynamic similarity is satisfied between the two pipes. The viscosity of water and oil is given as 0.01poise and 0.025poise. The specific gravity of oil is 0.8. |  | CO4 | L3 | 7M |
|  | b) | Give a neat sketch of a centrifugal pump showing all the important components and explain about its working. |  | CO4 | L2 | 7M |