20CE604

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August, 2023 | Civil Engineering |  |  |
| Sixth Semester | Advanced Structural Analysis |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | List any 2 factors influencing sway. | CO1 | L1 | 1 |
|  | b) | State the properties of flexibility matrix | CO1 | L2 | 1 |
|  | c) | A prismatic member is of length ‘L’ and has constant ‘EI’. At each end it has one vertical coordinate and a rotation coordinate. Obtain the flexibility matrix for the element. | CO1 | L1 | 1 |
|  | d) | What is static indeterminacy of a single bay single storey portal frame with vertical legs fixed at the base? | CO2 | L2 | 1 |
|  | e) | Compare flexibility method and stiffness method. | CO2 | L1 | 1 |
|  | f) | Sketch the stress-strain curve for a HYSD steel bar tested under uni-axial tension. | CO2 | L1 | 1 |
|  | g) | State upper bound theorem. | CO3 | L1 | 1 |
|  | h) | State Castiglianos theorem? | CO3 | L1 | 1 |
|  | i) | List the properties of a stiffness matrix? | CO4 | L2 | 1 |
|  | j) | What is shape factor? | CO4 | L1 | 1 |
|  | k) | What is rotation factor? | CO1 | L2 | 1 |
|  | l) | What do you mean by a pin jointed frame? | CO1 | L2 | 1 |
|  | m) | Express the relationship between flexibility and stiffness. | CO1 | L2 | 1 |
|  | n) | What is plastic moment? | CO2 | L2 | 1 |
| Unit-I |  |  |  |  |  |
| 2 |  | A Continuous beam ABCD 20 m long is continuous over 3 spans. AB = 8 m, BC = 4 m, CD = 8 m. Moment of Inertia of AB is 2I , that of BC is I and that of CD is 2I. There is a u.d.l. of 1500 N/m over spans AB & BC. On the span CD, there is a point load of 4000 N acting at 3 m from right end. Analyze the beam using Kanis method. Sketch B.M.D. Supports A and D are fixed. | CO1 | L4 | 14M |
|  |  | (OR) |  |  |  |
| 3 |  | Analyze the frame shown in fig.1 by Kanis method. Draw BMD. Fig.1 | CO1 | L4 | 14M |
| Unit-II |  |  |  |  |  |
| 4 |  | Analyse the continuous beam shown in the figure.2 using Strain energy method. Assume EI constant Sketch BMD. Fig.2 | CO2 | L4 | 14M |
| P.T.O 20CE604 (OR) |  |  |  |  |  |
| 5 |  | In the truss shown in figure. The member BD is initially shorter by 10 mm than its required length. Area = 1000 mm2 and E = 2X105 N/mm2 for all the members. Find the forces in all the members. fig.3 | CO2 | L4 | 14M |
| Unit-III |  |  |  |  |  |
| 6 |  | Analyse the continuous beam shown in figure.4 by flexibility method. Draw SFD and BMD. Fig.4 | CO3 | L4 | 14M |
| (OR) |  |  |  |  |  |
| 7 |  | Analyze the continuous beam shown in fig.5 by stiffness matrix method. Draw BMD. EI is constant. Fig.5 | CO3 | L4 | 14M |
| Unit-IV |  |  |  |  |  |
| 8 |  | Determine the collapse load for the beam shown in fig.6. Fig.6 | CO4 | L4 | 14M |
| (OR) |  |  |  |  |  |
| 9 | a) | Find the shape factor for T-section of flange 120 x 20 and web 180 x 20mm. | CO4 | L2 | 7M |
|  | b) | Find the shape factor for rectangular section of width B and depth D. | CO4 | L2 | 7M |