20CE605

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August,2023 | Civil Engineering |  |  |
| Sixth Semester | Estimation & Quantity Surveying |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  |  | CO | BL |  |  | M |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | a) |  | State the units of the following items : (i)Earthwork excavation (ii)Brick masonry | CO1 | L2 |  |  | 1M |  |  |
|  | b) |  | Define Quantity Surveying. | CO1 | L2 |  |  | 1M |  |  |
|  | c) |  | Calculate the quantity of brickwork in a semi-circular arch of 2.00 m span, thickness of the arch is 30 cm and the breadth of the wall is 40 cm. | CO1 | L2 |  |  | 1M |  |  |
|  | d) |  | Write the expression for calculating the length of a bent up bar cranked at both ends at 45°. | CO2 | L2 |  |  | 1M |  |  |
|  | e) |  | Express the formula for calculating the area of side sloping surface and expand the terms. | CO2 | L2 |  |  | 1M |  |  |
|  | f) |  | What do you mean by schedule of bars? | CO2 | L1 |  |  | 1M |  |  |
|  | g) |  | What are the factors affecting the rate of a particular item of work. | CO3 | L1 |  |  | 1M |  |  |
|  | h) |  | What do you mean by brief specification and detailed specification? | CO3 | L1 |  |  | 1M |  |  |
|  | i) |  | Define overhead costs. | CO3 | L2 |  |  | 1M |  |  |
|  | j) |  | What is plinth area estimate? | CO4 | L1 |  |  | 1M |  |  |
|  | k) |  | Why security money is needed? | CO4 | L1 |  |  | 1M |  |  |
|  | l) |  | Define the term obsolescence. | CO4 | L2 |  |  | 1M |  |  |
|  | m) |  | What do you mean by mortgage? | CO4 | L1 |  |  | 1M |  |  |
|  | n) |  | Define carpet area. | CO4 | L2 |  |  | 1M |  |  |
| Unit-I |  |  |  |  |  |  |  |  |  |  |
| 2 |  | Prepare an estimate by individual wall method for the following quantities from the given figure. (i) earthwork in excavation in foundation (ii) Lime concrete in foundation (iii) 1st class brick work in 1:6 cement Morata in foundation and plinth (iv) c.c damp proof course 2.5 cm (v) 1st class brick work in lime mortar in super structure. |  | CO1 | L5 |  | 14M |  |  |  |
| (OR) |  |  |  |  |  |  |  |  |  |  |
| 3 | a) | Write briefly about main items of work. |  | CO1 | L2 |  | 8M |  |  |  |
|  | b) | Write about the methods of building estimate. |  | CO1 | L2 |  | 6M |  |  |  |
| P.T.O 20CE605 Unit-II |  |  |  |  |  |  |  |  |  |  |
| 4 |  | Prepare a detailed estimate of a R.C.C. column with foundation footing. |  | CO2 | L5 |  | 14M |  |  |  |
| (OR) |  |  |  |  |  |  |  |  |  |  |
| 5 |  | Estimate the cost of earthwork for a portion of road for 400 metre length from the following data: Formation width of the road is 10 metre. Side slopes are 2:1 in banking and 1½:1 in cutting. Station; Distance (m); R.L. of Ground; R.L. of formation. 25; 1000; 51.00; 52.00 ǀ ǀ ǀ Downward gradient of 1 in 200 ǀ ǀ ǀ ǀ. 26; 1040; 50.90. 27; 1080; 50.50. 28; 1120; 50.80. 29; 1160; 50.60. 30; 1200; 50.70. 31; 1240; 51.20. 32; 1280; 51.40. 33; 1320; 51.30. 34; 1360; 51.00. 35; 1400; 50.60. | CO2 | L5 |  | 14M |  |  |  |
| Unit-III |  |  |  |  |  |  |  |  |  |  |
| 6 | a) | Explain the detailed specifications for II class Brick work. |  | CO3 |  | L2 |  | 7M |  |  |
|  | b) | Explain the detailed specifications for Mosaic flooring. |  | CO3 |  | L2 |  | 7M |  |  |
| (OR) |  |  |  |  |  |  |  |  |  |  |
| 7 | a) | Prepare rate analysis for R.C.C. work in column 1 : 1½ : 3 |  | CO3 |  | L5 |  | 7M |  |  |
|  | b) | Prepare rate analysis for Lime concrete in foundation with 40mm gauge Brick Ballast. |  | CO3 |  | L5 |  | 7M |  |  |
| Unit-IV |  |  |  |  |  |  |  |  |  |  |
| 8 | a) | Write a short note on - Gross Income - Net Income - Scrap value |  | CO4 |  | L2 |  | 6M |  |  |
|  | b) | Write in detail about the Measurement Book. |  | CO4 |  | L2 |  | 8M |  |  |
| (OR) |  |  |  |  |  |  |  |  |  | (OR) |
| 9 | a) | Discuss the organisation of engineering department. |  | CO4 |  | L2 |  | 8M |  |  |
|  | b) | Write a short note on - Capitalized value - Life of structure - Sinking fund. |  | CO4 |  | L2 |  | 6M |  |  |