20CE601

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August,2023 | Civil Engineering |  |  |
| Sixth Semester | Design of Steel Structures |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer one question from each unit. |  |  | (5X14=70 Marks) |
| Note:IS800:2007 & Steel Tables are allowed |  |  |  |

|  |  |  | CO | BL |  | M |
| --- | --- | --- | --- | --- | --- | --- |
| Unit-I |  |  |  |  |  |  |
| 1 | a) | Discuss the advantages of welded connections over bolted connections. | CO1 | L3 | 7M |  |
|  | b) | Explain the pitch requirements of bolts and define what is meant by efficiency of a joint? | CO1 | L3 | 7M |  |
|  |  | (OR) |  |  |  |  |
| 2 |  | A tie member of a truss consisting of an angle section ISA 65x65x6mm of Fe410 grade is welded to an 8mm gusset plate. Design a weld to transmit a load equal to full strength of the member. Assume shop welding. | CO1 | L5 | 14M |  |
| Unit-II |  |  |  |  |  |  |
| 3 |  | A single unequal angle ISA 100 x 60 x 6mm is connected to a 12mm gusset plates at the ends with 5no.of 16mm bolts to transfer tension. Bolt c/c distance is 50mm. Determine the design tensile strength of the angle, if the gusset is connected to 60mm leg. | CO2 | L4 | 14M |  |
| (OR) |  |  |  |  |  |  |
| 4 |  | Design a battened column with two channels back to back of length 8m to carry an axial load factored load of 1400KN. The column may be assumed to have restrained in position but not in direction at both ends. Assume Fe 410 grade. Use weld connection. | CO2 | L5 | 14M |  |
| Unit-III |  |  |  |  |  |  |
| 5 |  | Design a slab base under a column ISMB 300 carrying a factored load of 600 kN. The column rests on a pedestal of M20 grade concrete. | CO3 | L5 | 14M |  |
| (OR) |  |  |  |  |  |  |
| 6 |  | A column section ISHB300 at 63kg/m with one cover plate 400mm x 20mm on either side is carrying an axial load of 2000KN inclusive of self weight of base and column. Design a gusseted base for the column. The base rests on M15 grade concrete pedestal. | CO3 | L5 | 14M |  |
| Unit-IV |  |  |  |  |  |  |
| 7 |  | A simply supported beam of span 4m consists of a rolled steel section ISLB 325@422.8N/m. Determine the design bending strength of the beam if the beam is laterally unsupported. Assume that the factored shear force is less than the design shear strength. | CO4 | L4 | 14M |  |
| (OR) |  |  |  |  |  |  |
| 8 |  | Design a simply supported plated rolled steel beam section to carry a uniformly distributed load of 40 kN/m inclusive of self weight. Effective span of the beam is 5 m. The depth of the beam is not to exceed 450 mm. The compression flange of the beam is laterally supported. | CO4 | L5 | 14M |  |
| Unit-V |  |  |  |  |  |  |
| 9 |  | Design a seat connection for a factored beam end reaction of 150 kN. The beam section is ISMB 300 connected to the flange of column section ISHB 350 using bolted connections. Steel of grade Fe410 and bolts are of grade 4.6. | CO5 | L5 | 14M |  |
| (OR) |  |  |  |  |  |  |
| 10 |  | A bracket has to transmit an end reaction of 150KN to the column to which it is fillet welded on three sides as shown in figure. Design the joint as per I.S specifications. | CO5 | L5 | 14M |  |