18CED31

Hall Ticket Number:

| IV/IV B.Tech (Regular/Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| November, 2022 | Civil Engineering |  |  |
| Seventh Semester | Prestressed Concrete |  |  |
| Time: Three Hours |  | Maximum: 50 Marks |  |
| Answer Question No.1 compulsorily. |  |  | (10X1 = 10 Marks) |
| Answer ONE question from each unit. |  |  | (4X10=40 Marks) |

| 1. | a) |  | List the various strength requirements of steel used for prestressing systems. | CO1 | L1 | 1M |
| --- | --- | --- | --- | --- | --- | --- |
|  | b) |  | What are tendons? How are they used in prestressing systems? | CO1 | L2 | 1M |
|  | c) |  | What is pressure or thrust line? | CO1 | L3 | 1M |
|  | d) |  | How do you compute loss of stress due to shrinkage of concrete as per IS 1343: 2012? | CO2 | L2 | 1M |
|  | e) |  | Define is anchorage slip? | CO2 | L4 | 1M |
|  | f) |  | What are the factors that influence the deflections of prestressed concrete members? | CO2 | L4 | 1M |
|  | g) |  | What do you mean by failure of under reinforced sections? | CO3 | L2 | 1M |
|  | h) |  | Define effective reinforcement ratio? | CO3 | L3 | 1M |
|  | i) |  | How do you compute bond stress between hose and concrete? | CO4 | L2 | 1M |
|  | j) |  | How do you compute bursting tension in accordance with IS: 1343: 2012? | CO4 | L1 | 1M |
|  |  | Unit -I |  |  |  |  |
| 2. |  |  | What is the need of high strength and concrete in prestressing? Explain their advantages. | CO1 | L1 | 10M |
|  |  | (OR) |  |  |  |  |
| 3. |  |  | A rectangular concrete beam, 100 mm wide by 250 mm deep, spanning over 8 m is prestressed by a straight cable carrying an effective prestressing force of 200 kN located at an eccentricity of 40 mm. The beam supports a live load of 1.2 kN/m. (a) Calculate the resultant stress distribution for the central cross section of the beam. The density of concrete is 24 kN/m2. (b) Find the magnitude of the prestressing force with an eccentricity of 40 mm which can balance the stresses due to dead and live loads at the bottom fibre of the central section of the beam. | CO1 | L2 | 10M |
|  |  | Unit -II |  |  |  |  |
| 4. |  |  | Explain the various types of loss of prestress in pretensioned and post-tensioned members. | CO2 | L2 | 10M |
|  |  | (OR) |  |  |  |  |
| 5. |  |  | The deck of a prestressed concrete culvert is made up of a slab 500 mm thick. The slab is spanning over 10.4 m and supports a total uniformly distributed load comprising the dead and live loads of 23.5 kN/m2. The modulus of elasticity of concrete is 38 kN/mm2. The concrete slab is prestressed by straight cables each containing 12 high tensile wires of 7 mm diameter stressed to 1200 N/mm2 at a constant eccentricity of 195 mm. The cables are spaced at 328 mm intervals in the transverse direction. Estimate the instantaneous deflection of the slab at centre of span under prestress and the imposed loads. | CO2 | L3 | 10M |
| P.T.O 18CED31 Unit -III |  |  |  |  |  |  |
| 6. |  |  | A pretensioned, T-section has a flange 1200 mm wide and 150 mm thick. The width and depth of the rib are 300 and 1500 mm respectively. The high tensile steel has an area of 4700 mm2 and is located at an effective depth of 1600 mm. If the characteristic cube strength of the concrete and the tensile strength of steel are 40 and 1600 N/mm2, calculate the flexural strength of the T-section. | CO3 | L4 | 10M |
|  |  | (OR) |  |  |  |  |
| 7. |  |  | A prestressed girder of rectangular section 150 mm wide and 300 mm deep, is to be designed to support an ultimate shear force of 130 kN. The uniform prestress across the section is 5 N/mm2. Given the characteristic cube strength of concrete as 40 N/mm2 and Fe-415 HYSD bars of 8 mm diameter, design suitable spacing for the stirrups conforming to the Indian standard code IS: 1343 recommendations. Assume cover to the reinforcement as 50 mm. | CO3 | L2 | 10M |
|  |  | Unit -IV |  |  |  |  |
| 8. |  |  | A pretensioned beam, 160 mm wide by 320 mm deep, is prestressed by four plain wires of 7 mm diameter at an eccentricity of 100 mm. If the cube strength of concrete at transfer is 40 N/mm2, estimate the transmission length at the ends of the pretensioned units using IS: 1343 code provisions. | CO4 | L1 | 10M |
|  |  | (OR) |  |  |  |  |
| 9. |  |  | The end block of a post-tensioned beam is 80 mm wide and 160 mm deep. A prestressing wire, 7 mm in diameter, stressed to 1200 N/mm2 has to be anchored against the end block at the centre. The anchorage plate is 50 mm by 50 mm. The wire bears on the plate through a female cone of 20 mm diameter. Given the permissible stress in concrete at transfer, fci, as 20 N/mm2 and the permissible shear in steel as 94.5 N/mm2, determine the thickness of the anchorage plate. | CO4 | L3 | 10M |