20CS401

Hall Ticket Number:

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August,2023 | Computer Science and Engineering |  |  |
| Fourth Semester | Microprocessors & Microcontrollers |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL |  | M |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | a) | What is pipelining operation in 8086? | CO1 | L1 |  | 1M |
|  | b) | How does 8086 generate physical address? | CO1 | L3 |  | 1M |
|  | c) | What is Editor? | CO1 | L1 |  | 1M |
|  | d) | Define accumulator? | CO1 | L2 |  | 1M |
|  | e) | Explain MOVSB instruction in 8086. | CO2 | L2 |  | 1M |
|  | f) | What is Macro and when it is used? | CO2 | L2 |  | 1M |
|  | g) | Explain REP instruction of 8086. | CO2 | L2 |  | 1M |
|  | h) | What is the function of EQU directive in 8086? | CO2 | L1 |  | 1M |
|  | i) | What is the difference between minimum mode and maximum mode configurations of 8086. | CO3 | L1 |  | 1M |
|  | j) | Define Machine cycle. | CO3 | L1 |  | 1M |
|  | k) | Give software interrupt instruction of 8086. | CO3 | L2 |  | 1M |
|  | l) | What is embedded computer system? | CO4 | L1 |  | 1M |
|  | m) | Write any two single-bit instructions of 8051? | CO4 | L2 |  | 1M |
|  | n) | What is the difference between the Microprocessor and Microcontroller | CO4 | L2 |  | 1M |
| Unit-I |  |  |  |  |  |  |
| 2 | a) | Explain the register organization of 8086. | CO1 | L1 |  | 7M |
|  | b) | Discus the writing of 8086 program for use with an assembler. | CO2 | L2 |  | 7M |
|  |  | (OR) |  |  |  |  |
| 3 | a) | Write an 8086 assembly language program to multiply two 16 bit numbers. | CO2 | L3 |  | 7M |
|  | b) | What are the various instruction sets of 8086? Explain them in detail. | CO1 | L2 |  | 7M |
| Unit-II |  |  |  |  |  |  |
| 4 | a) | Explain various addressing modes of 8086. | CO2 | L1 |  | 7M |
|  | b) | Explain the following Assembler Directives i) SEGMENT ii) ASSUME iii) PROC iv) DW | CO1 | L1 |  | 7M |
| (OR) |  |  |  |  |  |  |
| 5 | a) | Explain Jump and call instructions of 8086. | CO2 | L2 | 7M |  |
|  | b) | Build an 8086 assembly language program to transfer a block of 50H data bytes stored in memory location from 4000H onwards to a memory location starting from 5000H using String instructions. | CO2 | L3 | 7M |  |
| Unit-III |  |  |  |  |  |  |
| 6 | a) | Demonstrate the memory write cycle operation of 8086 µp with a neat timing diagram. | CO3 | L3 |  | 4M |
|  | b) | Draw and explain the pin diagram of 8086 in minimum mode configuration. | CO1 | L2 |  | 10M |
| (OR) |  |  |  |  |  |  |
| 7 | a) | Draw and explain the interrupt structure of 8086. | CO3 | L2 |  | 7M |
|  | b) | Explain 8259A priority interrupt controller. | CO3 | L2 |  | 7M |
| Unit-IV |  |  |  |  |  |  |
| 8 | a) | Draw and explain the block diagram of 8051 microcontroller. | CO4 | L2 |  | 10M |
|  | b) | Write about addressing modes of 8051? | CO4 | L3 |  | 4M |
| (OR) |  |  |  |  |  |  |
| 9 | a) | Draw the pin diagram of 8051 and explain about I/O port pins. | CO4 | L1 |  | 10M |
|  | b) | Write the assembly code for the 8051 microcontroller to toggle the pins of PORT1. | CO4 | L5 |  | 4M |