20EC303

Hall Ticket Number:

| II/IV B. Tech (Regular / Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| February, 2023 | Electronics and Communication Engineering |  |  |
| Third Semester | Electronic devices and Circuits |  |  |
| Time: Three Hours |  | Maximum:70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

| 1. | a) |  |  | Compare diffusion capacitance and transition capacitance? | CO1 | L3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | b) |  |  | What is meant by LED? What materials are used to construct an LED? | CO1 | L1 |  |
|  | c) |  |  | What is tunnelling phenomenon? | CO1 | L1 |  |
|  | d) |  |  | Why an LC filter is called load independent? | CO2 | L1 |  |
|  | e) |  |  | Obtain the relationship between β and γ? | CO2 | L1 |  |
|  | f) |  |  | If the base current in a transistor is 50 micro amps when the emitter current is 2 m A. What are the values of α and β? | CO2 | L1 |  |
|  | g) |  |  | What are the differences between JFET and MOSFET? | CO2 | L1 |  |
|  | h) |  |  | What is meant by Pinch-off voltage? | CO3 | L1 |  |
|  | i) |  |  | Why SCR cannot be used as a bidirectional switch? | CO3 | L1 |  |
|  | j) |  |  | What is a DIAC? Give the basic construction and symbol of DIAC. | CO3 | L1 |  |
|  | k) |  |  | What is early effect | CO4 | L1 |  |
|  | l) |  |  | Draw the symbol of p-channel enhancement MOSFET | CO4 | L1 |  |
|  | m) |  |  | Draw the symbol of TRIAC | CO4 | L1 |  |
|  | n) |  |  | What is pinch-off voltage | CO4 | L1 |  |
|  |  | Unit-I |  |  |  |  |  |
| 2. | a) |  | Explain the current components in a PN junction diode, and derive the diode current equation. |  | CO1 | L2 | 7M |
|  | b) |  | Calculate the diffusion capacitance for a silicon diode with a10 mA forward current, if the charge carrier transit time is 60 nsec. |  | CO1 | L3 | 7M |
|  |  |  | (OR) |  |  |  |  |
| 3. | a) |  | Define tunneling phenomenon. With energy level diagram, Explain operation of tunnel diode under different operating conditions? |  | CO1 | L2 | 7M |
|  | b) |  | The reverse saturation of silicon PN junction diode is 20μA. Calculate the diode current for the forward bias voltage of 0.7V at 250C. |  | CO1 | L3 | 7M |
|  |  |  | Unit-II |  |  |  |  |
| 4. | a) |  | Explain the working of Bridge rectifier and derive the expressions for RMS current, PIV, ripple factor and efficiency. |  | CO2 | L2 | 7M |
|  | b) |  | A sinusoidal voltage whose Vm = 12V is applied to half-wave rectifier. The diode may be considered to be ideal and RL=2 KΩ is connected as load. Calculate IDC, IMAX, IRMS and γ. |  | CO2 | L3 | 7M |
|  |  |  | (OR) |  |  |  |  |
| 5. | a) |  | Explain the operation of L section filter with center tapped full wave rectifier? |  | CO2 | L2 | 7M |
|  | b) |  | A full wave rectifier circuit with capacitor filter is to supply a D.C. Current of 10 mA at 15V. If frequency is 50 Hz ripple allowed is 4%. Calculate C and the required secondary voltage of the transformer. |  | CO2 | L4 | 7M |
| P.T.O 20EC303 Unit-III |  |  |  |  |  |  |  |
| 6. | a) |  | Draw and explain the input and output characteristics of a transistor in CC configuration. |  | CO3 | L2 | 7M |
|  | b) |  | Determine the quiescent currents and the collector to emitter voltage for a Ge transistor with β = 50 in the self-biasing arrangements. The circuit component values are VCC = 20V, RC = 1kΩ, Re = 0.2 kΩ, R1 = 100 kΩ and R2 = 10 kΩ. Find the stability factor S. |  | CO3 | L4 | 7M |
|  |  |  | (OR) |  |  |  |  |
| 7. | a) |  | Explain the operation of Photo transistor? |  | CO3 | L2 | 7M |
|  | b) |  | A transistor is operated at a forward current of 2µA and with the collector open circuited. Calculate the junction voltages VC and VE, the collector to emitter voltage VCE assuming ICO = 2µA, IEO = 1.2µA and α = 0.96. |  | CO3 | L4 | 7M |
|  |  |  | Unit-IV |  |  |  |  |
| 8. | a) |  | Define and explain the three parameters of a JFET derive the relationship between them? |  | CO4 | L2 | 7M |
|  | b) |  | What is TRIAC? Sketch its characteristics and describe its operation? |  | CO4 | L4 | 7M |
|  |  |  | (OR) |  |  |  |  |
| 9. | a) |  | Explain the two-transistor model operation of SCR, and draw its V-I characteristics? |  | CO4 | L2 | 7M |
|  | b) |  | The reverse gate voltage of JFET, when changes from 4.2V to 4.0V, the drain current changes from 2.0 mA to 2.4 mA. The device parameters of JFET are maximum current IDSS = 8 mA, pinch off voltage, Vp = -45V. Find out the transconductance and drain current for VGS = -4V. |  | CO4 | L4 | 7M |