20EC305

Hall Ticket Number:

| II/IV B.Tech (Regular / Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| February, 2023 | Electronics and Communication Engineering |  |  |
| Third Semester | Digital Logic Design |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer Question No.1 compulsorily. |  |  | (14X1 = 14 Marks) |
| Answer ONE question from each unit. |  |  | (4X14=56 Marks) |

| 1. | a) |  |  | Compare Latch and Flip-flop. | CO1 | L3 | 1M |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | b) |  |  | Which flip flop is used as delay element? | CO1 | L1 | 1M |
|  | c) |  |  | Convert the SR flip-flop to D-flip-flop. | CO1 | L2 | 1M |
|  | d) |  |  | Write the condition for JK flip-flop to act as toggle only flip-flop. | CO1 | L1 | 1M |
|  | e) |  |  | List the contents of state table. | CO2 | L1 | 1M |
|  | f) |  |  | When the two states are said to be equivalent? | CO2 | L2 | 1M |
|  | g) |  |  | List all types of registers. | CO2 | L1 | 1M |
|  | h) |  |  | How many clock pulses are counted by 4-bit twisted ring counter? | CO2 | L2 | 1M |
|  | i) |  |  | Define propagation delay. | CO3 | L1 | 1M |
|  | j) |  |  | Compare TTL and CMOS logic. | CO3 | L3 | 1M |
|  | k) |  |  | Define fan-in and fan-out. | CO3 | L1 | 1M |
|  | l) |  |  | Write the size of decoder to design 64x8 ROM. | CO4 | L1 | 1M |
|  | m) |  |  | Compare PROM, PLA, PAL. | CO4 | L3 | 1M |
|  | n) |  |  | Compare volatile and non volatile memory. | CO4 | L4 | 1M |
|  |  | Unit - I |  |  |  |  |  |
| 2. | a) |  | Draw and explain the operation of SR flip flop using NAND gates |  | CO1 | L3 | 6M |
|  | b) |  | Convert the JK flip flop into T and D flip flop |  | CO1 | L4 | 8M |
|  |  | (OR) |  |  |  |  |  |
| 3. | a) |  | Draw and explain the operation of master slave JK flip-flop |  | CO1 | L2 | 8M |
|  | b) |  | Distinguish between combinational and sequential circuits |  | CO1 | L3 | 6M |
|  |  | Unit - II |  |  |  |  |  |
| 4. | a) |  | Draw and explain the operation of universal shift register |  | CO2 | L3 | 7M |
|  | b) |  | Design MOD-10 synchronous counter using T-Flip flops |  | CO2 | L3 | 7M |
|  |  | (OR) |  |  |  |  |  |
| 5. | a) |  | A sequential circuit has two JK flip-flops A and B and one input x . The circuit is described by the following flip-flop input equations: .Obtain the following i) State table. ii) State equations (iii) State diagram |  | CO2 | L3 | 8M |
|  | b) |  | Obtain the reduced state table and state diagram for the following: Present state; Next state X=0 X=1; Output. A; E; D; 0; 1. B; F; D; 0; 0. C; E; B; 0; 1. D; F; B; 0; 0. E; C; F; 0; 1. F; B; C; 0; 0. | CO2 | L3 | 6M |
|  |  | Unit - III |  |  |  |  |  |
| 6. | a) |  | Draw a CMOS 2 I/P NOR gate and verify its function table. |  | CO3 | L4 | 7M |
|  | b) |  | Compare various logic families |  | CO3 | L3 | 7M |
|  |  | (OR) |  |  |  |  |  |
| 7. | a) |  | Draw a CMOS 2 input NAND gate and verify its function table |  | CO3 | L2 | 7M |
|  | b) |  | Draw and explain the operation of ECL logic family |  | CO3 | L2 | 7M |
|  |  | Unit - IV |  |  |  |  |  |
| 8. | a) |  | Explain about the read and write operations of a RAM |  | CO4 | L2 | 6M |
|  | b) |  | Implement the following Boolean functions using PROM f1 (A, B, C) = ∑m (0, 1, 2, 4) ; f2 (A, B, C) = ∑m (0, 5, 6, 7) |  | CO4 | L3 | 8M |
|  |  | (OR) |  |  |  |  |  |
| 9. | a) |  | Write short notes on PROM,EPROM,EEPROM |  | CO4 | L2 | 6M |
|  | b) |  | Design a PLA circuit to implement the following functions: |  | CO4 | L4 | 8M |