20EC402

Hall Ticket Number:

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August, 2023 | Electronics and Communication Engineering |  |  |
| Fourth Semester | Electronic Circuit Analysis |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | List the typical h-parameter values of a transistor in different configurations. | CO1 | L1 | 1 |
|  | b) | Draw the small signal model of FET. | CO1 | L3 | 1 |
|  | c) | Draw the circuit of single stage CC transistor amplifier. | CO1 | L3 | 1 |
|  | d) | Why biasing is not required in push-pull amplifier | CO2 | L2 | 1 |
|  | e) | Why are multistage amplifiers preferred? | CO2 | L2 | 1 |
|  | f) | The turns ratio of transformer is 20:1. If a load of 10 ohms is connected across the primary, what will be effective resistance seen looking into secondary? | CO2 | L3 | 1 |
|  | g) | State any two advantages of negative feedback? | CO3 | L1 | 1 |
|  | h) | Draw the block diagram of Voltage shunt feedback amplifier. | CO3 | L2 | 1 |
|  | i) | Why oscillators exhibit positive feedback? | CO4 | L2 | 1 |
|  | j) | In Colpitts oscillator, C1= 0.16µH, L= 15.8mH and its frequency of oscillations is 10KHz, calculate the value of capacitor C2. | CO4 | L4 | 1 |
|  | k) | Give the classification of amplifiers | CO2 | L1 | 1 |
|  | l) | For a JFET amplifier, gm=2.5mA/V and rd=500kohms. The load resistance is 10Kohms. Find the voltage gain. | CO1 | L3 | 1 |
|  | m) | What is the main component that is responsible for the fall of gain of an RC Coupled amplifier in low frequency range? | CO2 | L1 | 1 |
|  | n) | What is an Oscillator? | CO2 | L2 | 1 |
| Unit-I |  |  |  |  |  |
| 2 | a) | A common collector amplifier with RS=1K ohm and RL=5 K ohm, hie=1.1kohms, hfe=50,hoe=25*10-6, hre=2.5*10-4. Evaluate voltage gain, current gain, input and output resistance using exact analysis of h-parameter model. | CO1 | L3 | 7M |
|  | b) | Analyze transistor Amplifier using h-parameters and find various gains and various impedances. | CO1 | L2 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | The transistor amplifier shown in figure uses a transistor with h- parameter values. Calculate Ai, AV, AVs, Ro and Ri. hie=1kohms, hfe=50,hoe=25*10-6, hre=2.5*10-4 and RS=2K. | CO1 | L3 | 7M |
|  | b) | Analyze CS amplifier along with circuit diagram using small signal model. | CO1 | L2 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | Explain the different types of coupling schemes used in multi stage amplifiers. | CO2 | L2 | 7M |
|  | b) | Draw the circuit of transformer coupled Class A power Amplifier and derive the expression for efficiency. | CO2 | L3 | 7M |
| P.T.O 20EC302 (OR) |  |  |  |  |  |
| 5 | a) | For a two stage amplifier shown in figure 1 calculate (a)Av (b) Avs, (c) Ri (d) Ro. Neglect the effect of all capacitances, Assume that both the transistors are identical with following parameters. hfe=50, hie=1.1KΩ, hre= 2.5 ×10-4 hoe=24×10-6 A/V | CO2 | L3 | 7M |
|  | b) | Discuss about the frequency response of an amplifier. | CO2 | L2 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | Draw the circuit diagram of current shunt feedback and derive expressions for input and output resistance | CO3 | L2 | 7M |
|  | b) | An RC coupled amplifier has a voltage gain of 1000, f1=50Hz,f2=200KHz and a distortion of 5% without feedback. Find the amplifier voltage gain, f1f, f2f and distortion when a negative feedback is applied with the feedback ratio of 0.01. | CO3 | L2 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | Enumerate the effects of negative feedback on the various characteristics of the amplifier? | CO3 | L2 | 7M |
|  | b) | Derive the expression for Rif and Rof in current series feedback amplifier? | CO3 | L3 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | State the Bark Hausen criteria. And explain about the operation of RC-Phase shift Oscillators. | CO4 | L2 | 7M |
|  | b) | Draw the circuit of Wein bridge oscillator and derive the expression for frequency of oscillations | CO4 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | List advantages and applications of crystal oscillator. A crystal oscillator has L=2H, C=0.01PF and R=2KΩ. Its mounting capacitance is 2PF. Calculate its series and parallel frequency? | CO4 | L2 | 7M |
|  | b) | Derive an expression for general form of LC Oscillators. | CO4 | L2 | 7M |