20EC503

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| February,2023 | Electronics and Communication Engineering |  |  |
| Fifth Semester | Digital Communications |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer Question No. 1 Compulsorily. |  |  | (14X1 = 14 Marks) |
| Answer ANY ONE question from each Unit. |  |  | (4X14=56 Marks) |

| 1. | a) | What is a Digital Signal? | CO2 | L2 | 1M |
| --- | --- | --- | --- | --- | --- |
|  | b) | Define Pulse width modulation. | CO1 | L1 | 1M |
|  | c) | Define Quantization error. | CO2 | L1 | 1M |
|  | d) | Differentiate Return to zero and non return to zero signal. | CO2 | L2 | 1M |
|  | e) | Write the properties of Matched filter. | CO3 | L3 | 1M |
|  | f) | Draw the block diagram of a correlation receiver. | CO2 | L2 | 1M |
|  | g) | Define modulation index. | CO2 | L2 | 1M |
|  | h) | Differentiate coherent and Non coherent detection. | CO2 | L2 | 1M |
|  | i) | Write the limitations of Binary Amplitude shift keying. | CO2 | L1 | 1M |
|  | j) | Write the equation for Probability of error of BFSK. | CO2 | L2 | 1M |
|  | k) | Define spread spectrum. | CO4 | L3 | 1M |
|  | l) | Write the relation between Information and probability of an event. | CO4 | L2 | 1M |
|  | m) | Define Information rate. | CO3 | L2 | 1M |
|  | n) | What are the minimum and Maximum values of entropy. | CO3 | L3 | 1M |
| Unit -I |  |  |  |  |  |
| 2. | a) | With a neat sketch explain the generation and reconstruction of Pulse Amplitude modulated signal. | CO1 | L1 | 7M |
|  | b) | In a single integration DM system, the voice signal is sampled at a rate of 64KHz, similar to PCM. The maximum signal amplitude is normalized as one. - Determine the minimum value of step size to avoid slope overload. - Determine the granular noise power if voice signal bandwidth is 3.4 KHz. | CO2 | L3 | 7M |
|  |  | (OR) |  |  |  |
| 3. | a) | Draw the block diagram of Differential Pulse Code Modulation and explain its operation, advantages and disadvantages. | CO2 | L3 | 7M |
|  | b) | Explain PCM with a neat block diagram. | CO2 | L2 | 7M |
|  |  | Unit -II |  |  |  |
| 4. | a) | What is correlative level coding and explain modified Duo-binary encoder. | CO3 | L1 | 7M |
|  | b) | Show that impulse response of matched filter is time reversed and delayed version of input signal | CO3 | L2 | 7M |
|  |  | (OR) |  |  |  |
| 5. | a) | What is Inter symbol Interference? Prove that ISI occurs in Base band transmission. | CO3 | L1 | 7M |
|  | b) | Explain in detail about Gram-Schmidt Orthogonaliizaation process. | CO2 | L1 | 7M |
|  |  | Unit -III |  |  |  |
| 6. | a) | Draw the block diagram of Frequency shift keying coherent demodulator and explain and derive the expression for Probability of error of FSK. | CO2 | L1 | 7M |
|  | b) | Discuss about coherent detection of Quadrature phase shit keying. | CO2 | L2 | 7M |
|  |  | (OR) |  |  |  |
| 7. | a) | Compare Binary ASK, FSK and PSK Modulation Techniques. | CO2 | L4 | 7M |
|  | b) | Explain BPSK and derive the expression for probability of error of binary PSK. | CO2 | L2 | 7M |
|  |  | Unit -IV |  |  |  |
| 8. | a) | What is spread spectrum communication and how dose it works. Describe the advantages of spread spectrum systems. | CO4 | L1 | 7M |
|  | b) | Explain the concept of Direct Sequence Spread Spectrum (DS-SS) and its system's performance parameters? | CO4 | L2 | 7M |
|  |  | (OR) |  |  |  |
| 9. | a) | A source generates 6 symbols with probabilities 0.5, 0.05, 0.05, 0.2, 0.1 , and 0.1 Find the efficiency when it is encoded using Shannon-Fano coding . | CO3 | L3 | 7M |
|  | b) | Define Entropy and Write its properties. | CO3 | L1 | 7M |