20EC603

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August, 2023 | Electronics & Communication Engineering |  |  |
| Sixth Semester | Digital Signal Processing |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. Butterworth and Chbyshev polynomial tables are allowed. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | What are the advantages of DSP? | CO1 | L1 | 1M |
|  | b) | Give the relationship between unit step sequence and unit impulse. | CO1 | L2 | 1M |
|  | c) | Evaluate the summation | CO1 | L2 | 1M |
|  | d) | Define LTI system. Give example. | CO1 | L2 | 1M |
|  | e) | State the Parseval’s property of DFS | CO2 | L2 | 1M |
|  | f) | What are the advantages of DFT over DTFT? | CO2 | L2 | 1M |
|  | g) | Determine the number of complex multiplications required to find the 32-point DFT of a sequence using FFT algorithm? | CO2 | L3 | 1M |
|  | h) | What are the advantages of Chebyshev filter over Butterworth filter? | CO3 | L2 | 1M |
|  | i) | What is the drawback of Impulse Invariance technique? | CO3 | L2 | 1M |
|  | j) | Convert analog filter with transfer function H(S) = into a digital filter by using bilinear transformation method with T= 1 sec. | CO3 | L3 | 1M |
|  | k) | Write the condition for symmetry with respect to the impulse response of a Linear phase FIR filter. | CO4 | L2 | 1M |
|  | l) | What is the advantage of Linear phase realization? | CO4 | L2 | 1M |
|  | m) | What are the applications of Multirate systems? | CO4 | L2 | 1M |
|  | n) | What is the purpose of a decimation filter? | CO4 | L2 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Derive the conditions for causality and stability of an LTI system. | CO1 | L3 | 7M |
|  | b) | Determine the convolution of following signals by means of Z-transform x(n) = u(n) and h(n) = 2nu(n) | CO1 | L4 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | State and prove any four properties of Z-transform. | CO1 | L3 | 7M |
|  | b) | Determine the step response of the following causal system described by the difference equation y(n) + 5y(n-1) + 6y(n-2) = x(n-1) | CO1 | L3 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | Determine the Fourier Series Coefficients of a signal x(n) = 1 + 2cos(πn/3) + 3 sin(πn/5) | CO2 | L3 | 7M |
|  | b) | Develop radix-2 Decimation-in-time FFT algorithm for N=8. | CO2 | L4 | 7M |
| (OR) |  |  |  |  |  |
| 5 | a) | Determine the linear convolution of the following signals using circular convolution. x(n) = {3, 1, 2, 1} and h(n) = {2, 1, 3} | CO2 | L3 | 7M |
|  | b) | Compute the 8-point DFT of the sequence x(n) = {1, 2, 3, 4, 4, 3, 2, 1} using radix-2 DIF FFT algorithm | CO2 | L4 | 7M |
| P.T.O 20EC603 Unit-III |  |  |  |  |  |
| 6 | a) | Obtain the impulse response of digital filter corresponding to an analog filter with impulse response ha(t) = 0.5e-2tu(t) and with a sampling rate of 1 kHz using Impulse Invariant method. | CO3 | L3 | 7M |
|  | b) | Derive the relationship between s-plane poles and z-plane poles in Bilinear transformation method. | CO3 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | Convert the analog filter with system function H(s) into a digital IIR filter by means of the impulse invariance method. H(s) = | CO3 | L3 | 7M |
|  | b) | Design a digital low pass filter is required to meet the following specifications. Pass band attenuation ≤ 1 db Stop band attenuation ≥ 40 db Pass band frequency = 4K Hz Stop band frequency = 8K Hz Sampling frequency = 24K Hz Use Butterworth approximation and bilinear transformation technique. | CO3 | L5 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Explain the principle and procedure for designing FIR digital filters using window method. | CO4 | L3 | 7M |
|  | b) | Obtain cascade form and Parallel form realizations for the system described by a difference equation y(n) = y(n-1) + y(n-2) + x(n) + 3 x(n-1) +2 x(n-2) | CO4 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | The desired response of a high pass filter is H(ejω) = e-j5ω ; π/4 ≤ ¦ω¦ ≤ π = 0 ; -π/4 ≤ ω ≤ π/4 Determine H(ejω) for N =11 using Hamming window. | CO4 | L4 | 7M |
|  | b) | Explain the concept of decimation and interpolation with examples. | CO4 | L3 | 7M |