20EE206

Hall Ticket Number:

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| August, 2023 | Electronics and Electronics Engineering |  |  |
| Third Semester | Engineering Mechanics |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | Two equal forces of magnitude P act at an angle 120°, their resultant will be i) P/2 ii) 2P iii) P iv) √2P | CO1 | L3 | 1M |
|  | b) | The centre of a semi-circle of radius R, represents origin. Express any feasible coordinate of the centroid | CO1 | L2 | 1M |
|  | c) | State Varignons theorem. | CO1 | L1 | 1M |
|  | d) | State parallel axis theorem. | CO2 | L1 | 1M |
|  | e) | The 100 N block is resting on horizontal surface. The coefficient of friction is 0.35. Minimum horizontal force required to create motion in the block is___________ | CO2 | L3 | 1M |
|  | f) | The friction force always acts in a direction opposite to i) the applied force ii) the motion or impending motion iii) both the force and the motion iv) neither the force nor the motion | CO2 | L1 | 1M |
|  | g) | Define curvilinear translation. | CO3 | L1 | 1M |
|  | h) | Write the equations of motion for the body moving with uniform acceleration. | CO3 | L1 | 1M |
|  | i) | D'Alembert's principle is used for i) Reducing the problem of kinetics to equivalent statics problem ii) Solving the problems of kinematics iii) Stability of floating bodies iv) Analyzing work-energy problems | CO3 | L1 | 1M |
|  | j) | Calculate the mass moment of inertia of a cone of mass 3kg, radius 40 mm and height 100 mm, about its axis. | CO4 | L3 | 1M |
|  | k) | Express the mass moment of inertia of a cylinder of mass ‘m’, diameter’d’ and height ‘h’, about its axis. | CO4 | L2 | 1M |
|  | l) | In fixed-axis rotation, all the particles have ____________ | CO4 | L1 | 1M |
|  | m) | Define angular displacement, angular velocity, angular acceleration. | CO4 | L1 | 1M |
|  | n) | Establish the relationship between angular motion and linear motion. | CO4 | L1 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Two smooth circular cylinders connected each other by a string AB of length 410 mm as shown in Fig. 1. Each of weight W = 450 N and radius r = 150 mm, supporting above them a third cylinder of weight Q = 890 N and radius r = 160 mm. Find the tension in the string and the reaction produced by the floor at the points of contact D and E. | CO1 | L3 | 14M |
|  |  | (OR) | P.T.O |  |  |
| 3 | a) | Locate the centroid of the shaded area shown in Fig. 2. Fig.2 | 20EE206 CO1 | L3 | 14M |
| Unit-II |  |  |  |  |  |
| 4 | a) | Determine the moments of inertia of the shaded area shown in Fig. 3 about its centroidal axes. | CO2 | L3 | 14M |
| (OR) |  |  |  |  |  |
| 5 | a) | Block A weighing 1000 N is to be raised by means of a 15° wedge B weighing 500 N as shown in Fig. 4. Assuming the coefficient of friction between all contact surfaces to be 0.28, find the minimum horizontal force P required to raise the block A Fig. 3 Fig. 4 | CO2 | L3 | 14M |
| Unit-III |  |  |  |  |  |
| 6 |  | A body weighing 1200 N rests on a rough plane inclined at 12ᵒ to the horizontal. It is pulled up the plane by means of a light flexible rope running parallel to the plane and passing over a light frictionless pulley at the top of the plane as shown in figure. The portion of the rope beyond the pulley hangs vertically down and carries a weight of 800 N at its end. If coefficient of friction for the plane and the body is 0.2, find: a) tension in the rope b) acceleration with which the body moves up the plane, and c) the distance moved by the body in 3 seconds after starting from rest. | CO3 | L3 | 14M |
| (OR) |  |  |  |  |  |
| 7 |  | A particle moves along a curvilinear path defined by y = ax2 where x and y are in meters. The velocity and acceleration of the particle at a point (5 m, 2.5 m) are respectively 5 m/s and 2 m/s2. Determine the total acceleration of the particle at that point. | CO3 | L3 | 14M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Derive the mass moment of inertia of a Solid Sphere with respect to centroidal axis. | CO4 | L3 | 14M |
| (OR) |  |  |  |  |  |
| 9 | a) | The angular acceleration of a flywheel is given by α = 12 – t, where α is in rad/sec2 and t is in seconds. If the angular velocity of the flywheelis 60 rad/sec at the end of 4 seconds. Determine the angular velocity at the end of 6 seconds. | CO4 | L3 | 7M |
|  | b) | Also find how many revolutions take place in these 6 seconds. | CO4 | L3 | 7M |