20EE405

Hall Ticket Number:

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August,2023 | Electrical and Electronics Engineering |  |  |
| Fourth Semester | Generation and Transmission |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | Define tariff. | CO1 | L1 | 1 |
|  | b) | List any three causes of low power factor? | CO1 | L1 | 1 |
|  | c) | Define plant use factor. | CO1 | L1 | 1 |
|  | d) | List any three disadvantages of a thermal power plant? | CO2 | L1 | 1 |
|  | e) | Define Nuclear Fission. | CO2 | L1 | 1 |
|  | f) | What are main Components in Wind Power plants. | CO2 | L1 | 1 |
|  | g) | Draw equivalent T network. | CO3 | L1 | 1 |
|  | h) | Define voltage regulation. | CO3 | L1 | 1 |
|  | i) | What is sag? | CO4 | L1 | 1 |
|  | j) | List any one advantage of corona loss. | CO4 | L1 | 1 |
|  | k) | Define string efficiency. | CO4 | L1 | 1 |
|  | l) | List the Types of power stations. | CO1 | L1 | 1 |
|  | m) | List the types of types of boilers in Thermal power plants. | CO2 | L1 | 1 |
|  | n) | Define GMD. | CO3 | L1 | 1 |
| Unit-I |  |  |  |  |  |
| 2 | a) | i) Explain different types of tariff’s. ii) A supply is offered on the basis of fixed charges of Rs 30 per annum plus 3 paise per unit or alternatively, at the rate of 6 paise per unit for the first 400 units per annum and 5 paise per unit for all the additional units. Find the number of units taken per annum for which the cost under the two tariffs becomes the same. | CO1 | L2 | 7M |
|  | b) | Discuss the various methods for power factor improvement | CO1 | L2 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | What do you understand by the load curve? What information’s are conveyed by a load curve ? | CO1 | L3 | 7M |
|  | b) | Explain the terms load factor and divesity factor. How do these factors influence the cost of generation? | CO1 | L2 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | Draw a general layout of a modern thermal power plant and explain the working of different circuits. | CO2 | L4 | 7M |
|  | b) | Explain the I-V and P-V characteristics of PV panels. | CO2 | L2 | 7M |
| (OR) |  |  |  |  |  |
| 5 | a) | Enumerate & explain essential components of a nuclear reactor | CO2 | L2 | 7M |
|  | b) | Draw a neat schematic diagram of a hydro-electric plant and explain the functions of various components. | CO2 | L4 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | Derive an expression for capacitances of a single phase transmission system and discuss the effect of earth on capacitance with suitable equation. | CO3 | L3 | 7M |
|  | b) | Derive expression for surge impedance. And also Explain the Ferranti effect in long transmission lines. | CO3 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | Derive the expression for ABCD parameters of medium transmission lines by π- method draw the vector diagram. | CO3 | L3 | 7M |
|  | b) | Determine inductance /km/phase of a single circuit 3-phase,20kv line shown below figure | CO3 | L4 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Discuss various methods for improving the string efficiency in a string of insulators. | CO4 | L3 | 7M |
|  | b) | Show how the sag of an overhead line can be calculated in case of supports at different levels. | CO4 | L2 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | Describe the phenomenon of corona? How can the corona loss is minimized in transmission lines. | CO4 | L2 | 7M |
|  | b) | Derive the expression for wave equation of a travelling wave. | CO4 | L4 | 7M |