20EE403

Hall Ticket Number:

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August,2023 | Electrical & Electronics Engineering. |
| Fourth Semester | Induction Motors & Synchronous Machines. |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | Define slip of an induction motor. | CO1 | L1 | 1M |
|  | b) | What is the condition for maximum running torque of a three phase induction motor? | CO1 | L1 | 1M |
|  | c) | What is the necessity of starter in three phase induction motors? | CO1 | L1 | 1M |
|  | d) | What is induction generator? | CO1 | L1 | 1M |
|  | e) | What is the role of centrifugal switch in a single phase induction motor? | CO2 | L1 | 1M |
|  | f) | How do you obtain the equivalent circuit of a single phase induction motor? | CO2 | L1 | 1M |
|  | g) | What are the applications of shaded pole motors? | CO2 | L1 | 1M |
|  | h) | Define voltage regulation of an alternator. | CO3 | L1 | 1M |
|  | i) | List the various conditions for successful parallel operation of alternators. | CO3 | L1 | 1M |
|  | j) | What is the effect of armature reaction at lagging power factor loads of an alternator? | CO3 | L1 | 1M |
|  | k) | Show the equivalent circuit of an alternator. | CO3 | L1 | 1M |
|  | l) | List out the various starting methods of synchronous motors. | CO4 | L1 | 1M |
|  | m) | What are the effects of hunting in a synchronous motor? | CO4 | L1 | 1M |
|  | n) | Draw the power flow diagram for synchronous motor. | CO4 | L3 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Explain the principle of operation of a three phase induction motor. | CO1 | L2 | 7M |
|  | b) | A three phase, 4 pole, 50 Hz induction motor has a full load speed of 1440 rpm. For this motor, calculate the following: - the synchronous speed - the full load slip and (c) rotor frequency. | CO1 | L3 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | Explain the DOL &Star-Delta Starting methods of 3-phase induction motor. | CO1 | L3 | 7M |
|  | b) | Explain the Various Speed control methods of 3-phase induction motor with neat sketches. | CO1 | L2 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | Explain why single phase induction motor is not self-starting. | CO2 | L2 | 7M |
|  | b) | Explain the equivalent circuit of a single phase induction motor. | CO2 | L2 | 7M |
| (OR) |  |  |  |  |  |
| 5 | a) | Explain the resistance phase splitting method of starting of a single phase induction motor. | CO2 | L2 | 7M |
|  | b) | Explain the construction and principle of operation of a shaded pole motor. | CO2 | L2 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | Derive the emf equation of an alternator when the winding is distributed and short pitched. | CO3 | L3 | 7M |
|  | b) | A three phase, 10 pole, star connected alternator runs at 600 rpm. It has 120 slots with 8 conductors per slot and conductors of each phase are connected in series. The armature winding short pitched by 2 slots. Determine the phase and line electromotive forces if the flux per pole is 56 mWb. | CO3 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | Explain Two-reaction theory of salient pole synchronous generator with a neat Sketch. | CO3 | L3 | 7M |
|  | b) | Explain any method to synchronize the alternator to an infinite busbar. | CO3 | L2 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Explain the principle of operation of a synchronous motor with neat diagrams. | CO4 | L2 | 7M |
|  | b) | A 2300 V three phase star connected synchronous motor has synchronous impedance of (0.2+j2.2) Ω/phase. The motor is operating at 0.5 power factor leading with line current of 200 A. Determine back emf per phase. | CO4 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | Explain the concepts of V & Inverted V curves of synchronous motor with neat diagrams. | CO4 | L2 | 7M |
|  | b) | What is synchronous condenser? Explain the use of synchronous condenser. | CO4 | L2 | 7M |