20EI602

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August,2023 | Electronics and Instrumentation Engineering |  |  |
| Sixth Semester | Digital Signal Processing |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | Check whether the following system is causal. y(n) = 2x(n) + 3u(n + 1) | CO1 | L2 | 1M |
|  | b) | Write expressions for system function and transfer function. | CO1 | L1 | 1M |
|  | c) | What are the applications of Z-Transforms? | CO1 | L1 | 1M |
|  | d) | Define DFT and IDFT. | CO2 | L1 | 1M |
|  | e) | Write the relation between DFS and DFT. | CO2 | L2 | 1M |
|  | f) | What do you mean by Canonical form of Realization? | CO2 | L2 | 1M |
|  | g) | List out the characteristic features of FIR filters. | CO3 | L2 | 1M |
|  | h) | What is the necessary and sufficient condition for linear phase Characteristics of an FIR filter? | CO3 | L3 | 1M |
|  | i) | List out the methods used for design of digital IIR filters? | CO4 | L1 | 1M |
|  | j) | Compare IIR and FIR filters. | CO4 | L1 | 1M |
|  | k) | Find the Z transform of {1,0,2,0,3} | CO1 | L2 | 1M |
|  | l) | What is the Z-transform of ‘nx(n)’ | CO1 | L3 | 1M |
|  | m) | What is frequency wrapping and how it can be eliminated? | CO1 | L2 | 1M |
|  | n) | State and prove periodicity property of DFT. | CO2 | L2 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | A causal LTI system is described by the Linear constant coefficient difference equation (LCCD), y(n)=0.75 y(n-1)-0.125 y(n-2)+x(n). Determine impulse and step responses of the system | CO1 | L3 | 7M |
|  | b) | Determine whether the following system is: i) Linear ii) Causal iii) Stable, and iv) Time invariant. y (n)=log10 ¦x (n) ¦ Justify your answer. | CO1 | L2 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | What is the significance of convolution? Explain | CO1 | L2 | 7M |
|  | b) | Determine the impulse response of the following causal system with difference equation y(n) = 0.6y(n-1) - 0.08y(n-2) + 2x(n) | CO1 | L3 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | Determine the Fourier series coefficient of the signal x(n) and plot the magnitude and phase spectra | CO2 | L3 | 7M |
|  | b) | Compute the eight point DFT of the sequence x= [1,1,1,1,1,1,1,1], using Decimation-in Frequency FFT algorithm. | CO2 | L4 | 7M |
| (OR) |  |  |  |  |  |
| 5 | a) | Compute the eight-point DFT of the sequence Using the in-place radix-2 decimation-in-time algorithm. Follow exactly the corresponding signal flow graphs and keep track of all the intermediate quantities by putting them on the diagrams. | CO2 | L4 | 7M |
|  | b) | Describe the following properties of DFT. i. Time reversal ii. Circular convolution. | CO2 | L3 | 7M |
| P.T.O 20EI602 Unit-III |  |  |  |  |  |
| 6 | a) | What is a Kaiser window? In what way is it superior to other window functions? | CO3 | L2 | 7M |
|  | b) | Using frequency sampling method design a BPF with the following specifications: (i) Sampling frequency (ii) Cutoff frequencies and | CO3 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | Obtain FIR linear-phase realisation of the system function | CO3 | L3 | 7M |
|  | b) | The desired frequency response of a low pass filter is Determine hd (n) for M = 7 using a rectangular window. | CO3 | L4 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Explain the method of design of IIR filters using bilinear transform method. | CO4 | L2 | 7M |
|  | b) | Design a second order digital low pass Butterworth filter with a cut-off frequency 3.4 KHz at a sampling rate of 8 KHz using bilinear transformation. | CO4 | L4 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | Explain impulse invariant method of designing IIR filter. | CO4 | L2 | 7M |
|  | b) | Obtain cascade and parallel form realizations of the system with system function | CO4 | L3 | 7M |