18EII04

Hall Ticket Number:

| IV/IV B.Tech (Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| November, 2022 | Institutional Elective (Common to all branches) |  |  |
| Eighth Semester | Sensors and Signal Conditioning |  |  |
| Time: Three Hours |  | Maximum: 50 Marks |  |
| Answer Question No.1 compulsorily. |  |  | (10X1 = 10 Marks) |
| Answer ONE question from each unit. |  |  | (4X10=40 Marks) |

| 1 | a) |  | Give two examples for active and passive transducers. |  | CO1,L1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | b) |  | Differentiate the terms range and span. |  | CO1,L2 |  |  |  |
|  | c) |  | Give the advantages of thermistor. |  | CO1,L1 |  |  |  |
|  | d) |  | What is residual voltage in LVDT? |  | CO2,L1 |  |  |  |
|  | e) |  | Define Seebeck effect. |  | CO2,L1 |  |  |  |
|  | f) |  | What does a transimpedance amplifier do? |  | CO3,L1 |  |  |  |
|  | g) |  | What are the suitable materials for piezo electric transducer? |  | CO3,L1 |  |  |  |
|  | h) |  | What is the need of a charge amplifier? |  | CO1,L1 |  |  |  |
|  | i) |  | Define CMRR. |  | CO2,L1 |  |  |  |
|  | j) |  | List out the components of an intelligent sensor. |  | CO1,L1 |  |  |  |
|  |  | Unit -I |  |  |  |  |  |  |
| 2 | a) |  | Describe the classification of transducers along with examples of each case. |  |  | CO1,L3 |  | 5M |
|  | b) |  | An instrument has an accuracy of ± 0.5% FS and measures resistance from 0 to 1500 Ω. What is the resistance range in an indicated measurement of 397 Ω. Find the accuracy in the measured value of resistance R. |  |  | CO1,L3 |  | 5M |
|  |  |  | (OR) |  |  |  |  |  |
| 3 | a) |  | Discuss signal conditioning for resistive transducers using Wheatstone bridge in the deflection mode. |  |  | CO1,L4 |  | 5M |
|  | b) |  | A metallic strain gauge has a resistance of 120Ω & a gauge factor of 2. It is installed on an aluminium structure which has a yield point stress of 0.2 GN/m2 & Young's modulus of 68.7 GN/m2. Determine the change in resistance (in Ω) of the gauge that would be caused by loading the material to yield point. |  |  | CO1,L3 |  | 5M |
|  |  |  | Unit -II |  |  |  |  |  |
| 4 | a) |  | Draw the schematic of " Liquid metal level measurement based on eddy currents" and explain its working principle. |  |  | CO2,L3 |  | 5M |
|  | b) |  | Draw the circuit of carrier amplifier and explain the need of it. |  |  | CO2,L2 |  | 5M |
|  |  |  | (OR) |  |  |  |  |  |
| 5 | a) |  | Explain how LVDT can be used for pressure measurement? |  |  | CO1,L2 |  | 5M |
|  | b) |  | Discuss charge redistribution method of signal conditioning for capacitive sensors. |  |  | CO1,L2 |  | 5M |
|  |  |  |  | Unit -III |  |  |  |  |
| 6 | a) |  | A copper constantan thermocouple was found to have linear calibration between 0oC and 400oC with emf at maximum temperature (w.r.t the cold junction temperature of 0oC) being equal to 20 mV.(i)Determine the correction which must be made to the indicated emf if the cold junction temperature is 20oC.(ii)If the indicated emf is 8.90 mV (w.r.t the cold junction temperature at 20oC) in the thermocouple circuit, determine the temperature of the hot junction. |  |  | CO3,L2 |  | 5M |
|  | b) |  | Illustrate different possible low-frequency applications for the piezoelectric effect. |  |  | CO3,L2 |  | 5M |
|  |  |  | (OR) |  |  |  |  |  |
| 7 | a) |  | Explain the working principle of pyro-electric sensors with necessary equations. |  |  | CO3,L2 |  | 5M |
|  | b) |  | Draw the circuit of Instrumentation amplifier and write the expression for output voltage. |  |  | CO3,L2 |  | 5M |
|  |  |  | Unit -IV |  |  |  |  |  |
| 8 | a) |  | Find Draw the diagram of absolute position encoder and explain its operation. |  |  | CO4,L2 |  | 5M |
|  | b) |  | sh Write the expression for frequency in a basic Wien bridge oscillator with a neat diagram. |  |  | CO4,L2 |  | 5M |
|  |  |  | (OR) |  |  |  |  |  |
| 9 | a) |  | Draw the circuit of a voltage-to-frequency converter based on the charge balancing technique and explain its operation. |  |  | CO4,L3 |  | 5M |
|  | b) |  | A given sensor has an output frequency of 9 kHz to 11 kHz. Determine the measurement time and the number of counts needed to measure the frequency with a 12 bit resolution. |  |  | CO4,L3 |  | 5M |