| 20ME104 Hall Ticket Number: I/IV B.Tech(Regular/ Supplementary) DEGREE EXAMINATION. March,2023; Mechanical Engineering. First Semester; Engineering Mechanics - I. Time: Three Hours; Maximum:70 Marks. |  |
| --- | --- |
| Answer question 1 compulsory.; (14X1 = 14 Marks). Answer one question from each unit.; (4X14 = 56 Marks). |  |

| 1. | a) |  | State Lami’s theorem. | CO1 | L1 | 1M |
| --- | --- | --- | --- | --- | --- | --- |
|  | b) |  | What is free body diagram and state its importance? | CO1 | L1 | 1M |
|  | c) |  | Define resultant of a force system. | CO1 | L1 | 1M |
|  | d) |  | State Varignon’s theorem. | CO1 | L1 | 1M |
|  | e) |  | Distinguish moment of a force and a couple. | CO1 | L3 | 1M |
|  | f) |  | What is perfect truss? How do you check the perfectness of a truss? | CO2 | L1 | 1M |
|  | g) |  | State laws of dry friction. | CO2 | L1 | 1M |
|  | h) |  | Define angle of friction. | CO2 | L1 | 1M |
|  | i) |  | Define centroid. | CO3 | L1 | 1M |
|  | j) |  | Write the expressions for centroid of composite plane figure. | CO3 | L1 | 1M |
|  | k) |  | State Pappus theorem-I. | CO3 | L2 | 1M |
|  | l) |  | Define polar moment of inertia. | CO4 | L1 | 1M |
|  | m) |  | Define radius of gyration. | CO4 | L1 | 1M |
|  | n) |  | Write the formula for the moment of inertia of a circle of radius ‘R’ about its diametric axes. | CO4 | L3 | 1M |
|  |  | Unit –I |  |  |  |  |  |  |
| 2. | a) |  | A system of four forces acting on a body is as shown in figure. Determine the magnitude and direction of resultant. | CO1 | L2 | 6M |
|  | b) |  | Two identical rollers, each of weight Q = 1000 N are supported by an inclined plane and a vertical wall as shown in figure. Assuming smooth surfaces, find the reactions induced at the points of support A, B and C. | CO1 | L3 | 8M |
|  |  | (OR) |  |  |  |  |
| 3. | Three beams hinged together at their ends are supported and loaded as shown in figure. Determine the reactions at the supports A, B, C and D, if Q = 4450 N. |  |  | CO1 | L4 | 14M |
| P.T.O 20ME104 Unit –II |  |  |  |  |  |  |
| 4. | A pin jointed truss is loaded and supported as shown in figure. Determine forces in all the members. |  |  | CO2 | L2 | 14M |
|  |  | (OR) |  |  |  |  |
| 5. | A block of weight W1 = 1000 N rests on a horizontal surface and supports on its top another block of weight W2 = 250 N as shown in figure. The weight W2 is attached by an inclined string AB to the vertical Wall. Find the magnitude of the horizontal force P applied to the lower block to cause slipping to impend. The coefficient of friction for all contacting surfaces may be assumed to µ = 0.3. |  |  | CO2 | L4 | 14M |
|  |  | Unit –III |  |  |  |  |
| 6. | A semicircular area is removed from the trapezoid shown in figure. Determine the coordinates of the centroid for the shaded area. |  |  | CO3 | L3 | 14M |
|  |  | (OR) |  |  |  |  |
| 7. | Determine the coordinates of centroid of the area bounded by the parabola y = kx2, the straight-line x = a and x-axis as shown in figure. |  |  | CO3 | L2 | 14M |
|  |  | Unit –IV |  |  |  |  |
| 8. | a) |  | State and prove parallel axis theorem. | CO4 | L3 | 7M |
|  | b) |  | Derive the formula for moment of inertia of a triangle of base ‘b’ and height ‘h’ about its base. | CO4 | L2 | 7M |
|  |  | (OR) |  |  |  |  |
| 9. | Find moment of inertia of the shaded area shown in the figure about the axis passing through and perpendicular to AB and having the origin at A. |  |  | CO4 | L3 | 14M |