20CE503

Hall Ticket Number:

| III/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| December, 2023 | Civil Engineering |  |  |
| Fifth Semester | Design of Concrete Structures |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Note : 1) Use of IS 456:2000 code book is permitted. 2) Assume suitable data wherever necessary. |  |  |  |
| Answer ONE question from each unit. |  |  | (5X14=70 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| Unit-I |  |  |  |  |  |
| 1 | a) | Discuss about the partial safety factors for material strength in limit state of collapse and in limit state of serviceability? | CO1 | L2 | 7M |
|  | b) | What does it mean by ‘balanced section’ as per Working Stress Method and Limit State Method? Explain with neat sketches. | CO1 | L3 | 7M |
|  |  | (OR) |  |  |  |
| 2 | a) | Discuss the various applications of reinforced concrete in structural members. | CO1 | L3 | 7M |
|  | b) | A RCC beam with 250mm x 400mm is provided with 4 nos 25mm dia bars in tension. M30 grade concrete and Fe 415 steel are used. Take 40mm effective cover. Calculate moment of resistance and percentage area of steel if the beam is a balanced section. | CO1 | L4 | 7M |
| Unit-II |  |  |  |  |  |
| 3 | a) | The provision of minimum stirrup reinforcement is mandatory in all reinforced concrete beams. Why? | CO2 | L3 | 7M |
|  | b) | A simply supported beam is 250mm x 500mm and has 2 nos – 20mm going into the support. If the shear force at the centre of supports is 110kN at working loads, Determine the development length. Assume M20 grade concrete and Fe 415 steel. | CO2 | L4 | 7M |
| (OR) |  |  |  |  |  |
| 4 |  | Design shear reinforcement for an RC beam of size 350mm wide and having an effective depth of 550 mm, and carrying a live load of 4 kN/m throughout the length. Use M25 concrete and Fe 415 grade steel. The percentage of tension steel is 1.67. Sketch the reinforcement details. | CO2 | L5 | 14M |
| Unit-III |  |  |  |  |  |
| 5 |  | Design a simply supported slab to cover a hall with internal dimensions 4m x 6m. The slab is supported on masonry walls 230mm thick. Assume a live load of 3kN/m2 and a finish load of 1 kN/m2. Use M30 concrete and Fe 415 steel. Assume that the slab corners are prevented from lifting up. | CO3 | L5 | 14M |
| (OR) |  |  |  |  |  |
| 6 |  | A hall has clear dimension 3m x 9m with wall thickness 230mm. The live load on the slab is 3kN/m2 and a finishing load of 1kN/m2 may be assumed. Using M25 grade concrete and Fe 415 grade steel, design the one-way slab. | CO3 | L5 | 14M |
| Unit-IV |  |  |  |  |  |
| 7 | a) | What are braced and unbraced columns? What parameter in column design is affected by bracing? | CO4 | L3 | 6M |
|  | b) | A short rectangular column 5m is to carry a characteristic load of 300kN. Assume the ends of the column are fully restrained. Design column size, longitudinal reinforcement, and lateral ties. Use M25 concrete and Fe 415 steel. | CO4 | L5 | 8M |
| P.T.O 20CE503 |  |  |  |  |  |
| (OR) |  |  |  |  |  |
| 8 | a) | Explain the function of transverse ties in a reinforced concrete column? What happens if ties are not provided. | CO4 | L3 | 6M |
|  | b) | A short column 400mm x 400mm is reinforced with 4nos-25mm diameter bars longitudinally, which are bound together with lateral ties. Determine the safe axial load on the column. Use M30 concrete and Fe 415 steel. | CO4 | L4 | 8M |
| Unit-V |  |  |  |  |  |
| 9 |  | Design a square footing for a circular column 500mm in diameter carrying an axial load of 2500kN. The safe bearing capacity of soil is 300kN/m2 at a depth of 1.5m below ground. Use Fe 415 steel and M25 concrete. Sketch the details of reinforcement. | CO5 | L5 | 14M |
| (OR) |  |  |  |  |  |
| 10 |  | Design a square footing for a short axially loaded column of size 300mm x 300mm carrying 600kN load. Use M30 concrete and Fe 415 steel. Safe Bearing Capacity of soil is 180kN/m2. Sketch the details of reinforcement. | CO5 | L5 | 14M |