20CE705/JO

Hall Ticket Number:

| IV/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2024 | Civil Engineering |  |  |
| Seventh Semester | Bridge Engineering |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer ONE question form each Unit. |  |  | (5X14=70 Marks) |
| IS: 456, IRC-6 and IRC-21 code books are allowed to the examination. Assume any relevant data if necessary. |  |  |  |

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | What is an economical span? Derive expression for an economical span. | CO1 L2 | 4M |
|  | b) | Write about choice of bridge type by stating their suitability for different conditions. | CO1 L2 | 10M |
| (OR) |  |  |  |  |
| 2. |  | Explain in brief about selection of bridge site. | CO1 L2 | 14M |
| UNIT-II |  |  |  |  |
| 3. |  | Design a Deck slab for a culvert on state highway for Class AA Tracked vehicle. Width of bridge is 12m. The materials used for concrete is M25 and that of steel is Fe415. The clear span of bridge is 5.5m and wearing course of 56mm thick asphaltic concrete. | CO2 L3 | 14M |
| (OR) |  |  |  |  |
| 4. |  | Explain in detail about IRC standard live loads with sketches. | CO2 L2 | 14M |
| UNIT-III |  |  |  |  |
| 5. |  | Design a cantilever slab for a T-Beam bridge having a span of 15m on a state highway having an overhang of 1.5m from the end longitudinal girder. Assume M25 grade concrete and Fe415 steel with moderate exposure condition. | CO3 L3 | 14M |
| (OR) |  |  |  |  |
| 6. |  | A T-Beam bridge is to be built with three longitudinal girders and five cross girders on a rural section of a state highway, having a span of 16m. Design the intermediate longitudinal girder with class A loading. Assume M25 grade concrete and Fe415 steel with moderate exposure condition. | CO3 L3 | 14M |
| UNIT-IV |  |  |  |  |
| 7. |  | Find the summary of stresses on pier and check the adequacy of dimensions. Superstructure: Simply supported T-Beam of 21.3m span Foundation: well foundation Dead load from each span = 2250 kN Reaction due to live load on one span = 900 kN Maximum mean velocity of current = 3.6 m/sec. Material for pier: Cement concrete M20 grade. Live load: IRC Class AA Only the straight portion of pier will be considered in design here. Assume relevant dimensions. | CO4 L3 | 14M |
| (OR) |  |  |  |  |
| 8. |  | Find the summary of forces and moments of abutment and check the adequacy of dimensions. Superstructure: T-beam two-lane bridge of effective span 16.1m Overall length = 17.26m Type of abutment: Reinforced concrete Loading: As for national Highway Back fill: Gravel with angle of repose = 35o Unit weight of back fill, w = 18 kN/m3 Angle of internal friction of soil on wall, Z=17.5 degrees Approach slab: R.C slab 300mm thick, adequately reinforced Load from superstructure per running meter of abutment: Dead load = 119 kN/m, Live load = 85 kN/m. Bearings: Neoprene pads of overall size 320x500x65mm, embedding 5 plates of 3 mm thickness and 6 mm clearance in plan. G = 1 kN/mm2 . | CO4 L3 | 14M |
| UNIT-V |  |  |  |  |
| 9. |  | Explain in detail with neat sketches of types of bearings for Girder bridges. | CO5 L2 | 14M |
| (OR) |  |  |  |  |
| 10. |  | Design an elastometric pad bearing for a two-lane R.C T-beam bridge of 15m clear span with the following data: Maximum dead load reaction per bearing = 280kN Maximum live load reaction per bearing = 520kN Vertical reaction induced by longitudinal forces per bearing = 12kN Longitudinal force per bearing = 33kN A1/A2 >2, Use M20 grade concrete. Rotation at bearing of superstructure due to live load and dead load = 0.0025 radian. | CO5 L3 | 14M |