Hall Ticket Number:                                                                                                     20EE304

| II B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2024 | Electrical and Electronics Engineering |  |  |
| Third Semester | DC Machines and Transformers |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | Define reluctance? | CO1 | L1 | 1M |
|  | b) | State Flemings right hand thumb rule? | CO1 | L1 | 1M |
|  | c) | State Ampere circuit law? | CO1 | L1 | 1M |
|  | d) | State Faradays First Law? | CO2 | L1 | 1M |
|  | e) | Write the number of parallel paths for Lap and Wave winding? | CO2 | L1 | 1M |
|  | f) | List the types of self-excited DC Generators? | CO2 | L1 | 1M |
|  | g) | State Flemings left hand rule? | CO3 | L1 | 1M |
|  | h) | What happens if DC motor run without starter? | CO3 | L1 | 1M |
|  | i) | Which is the indirect method of testing DC machine? | CO3 | L1 | 1M |
|  | j) | Write the types of Iron losses? | CO4 | L1 | 1M |
|  | k) | Define Voltage regulation? | CO4 | L1 | 1M |
|  | l) | Draw the phasor diagram of transformer for Inductive load? | CO4 | L1 | 1M |
|  | m) | Write any two applications of DC Generator? | CO2 | L1 | 1M |
|  | n) | Draw the equivalent circuit of transformer? | CO3 | L1 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Derive the expression of field energy and the magnetic force in a singly excited electromagnetic unit. | CO1 | L2 | 7M |
|  | b) | A ring has a diameter of 24cm and a cross sectional area of 1000 mm2. the ring is made up of semicircular section of cast iron and cast steel with each joint having a reluctance equal to an air-gap of 0.2 mm. Find the ampere turns required to produce a flux of 8×10-4 wb. The relative permeability of cast steel and cast iron are 900 and 170 respectively. Neglect fringing and leakage effects | CO1 | L2 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | Discuss in detail the production of mechanical force for an attracted single excited armature relay excited by an electric source? | CO1 | L2 | 7M |
|  | b) | Examine the typical B-H Curve and hysteresis loops with neat sketch. | CO1 | L1 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | Explain the principle and operation of DC Generator? | CO2 | L1 | 7M |
|  | b) | A separately excited DC generator has armature circuit resistance of 0.22 Ω and a total drop at brushes is 2.5 V. When running at 1200 rpm, it delivers a current of 120 A at 220 V to a load of constant resistance If the generator speed drops to 750 rpm, with field current unaltered, find the current delivered to load. | CO2 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 5 | a) | Sketch and explain the load characteristics of the shunt and series Generator? | CO2 | L2 | 7M |
|  | b) | Illustrate the process of commutation in DC Generator? | CO2 | L2 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | Derive the Torque equation of DC motor? | CO3 | L1 | 7M |
|  | b) | A 200 V DC shunt motor with an armature resistance of 0.3 Ω is excited to give constant main field with field resistance of 0.02 Ω. At full load the motor runs at 600 rpm and takes an armature current of 30 A. If a resistance of 1 Ω is placed in the armature circuit, find the speeds at full load torque and twice the full load torque. | CO3 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | Illustrate the characteristics of the following DC motors i. DC Shunt Motor ii. DC Series motor | CO3 | L1 | 7M |
|  | b) | A 220 V, DC shunt motor is operating at a speed of 1440 rpm. The armature resistance is 1.0 Ω, field resistance of 0.3 Ω and armature current is 10 A. If the excitation of the machine is reduced by 10%, then find the value of extra resistance to be put in the armature circuit to maintain the same speed and torque. | CO3 | L3 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Explain about the short circuit test of a single phase transformer and give its significance? | CO4 | L1 | 7M |
|  | b) | A single phase transformer is rated at 120 kVA, 5000/250V. The full – load copper losses are 2200W and iron losses are 1400 W. Find efficiency at i) full – load 0.8 power factor leading ii) full – load 0.6 power factor lagging | CO4 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | Discuss the types of connections and features in three phase transformers? | CO4 | L2 | 7M |
|  | b) | A single phase transformer with a ratio of 440/110V takes a no-load current of 5A at 0.2 p.f lagging. If the secondary draws a current of 120A on load at 0.8 p.f lagging, calculate the primary current I1 in primary? Draw the phasor diagram? | CO4 | L3 | 7M |