20EE303

Hall Ticket Number:

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2024 | Electrical &Electronics Engineering |  |  |
| Third Semester | Electromagnetic Fields |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | Convert the point P(2,6,-4) into cylindrical coordinates | CO1 | L3 | 1M |
|  | b) | Name the various electric charge distributions. | CO1 | L2 | 1M |
|  | c) | State Gauss Law. | CO1 | L2 | 1M |
|  | d) | What is potential gradient? | CO2 | L2 | 1M |
|  | e) | Define Electric Dipole. | CO2 | L2 | 1M |
|  | f) | Distinguish between convection and conduction currents. | CO2 | L3 | 1M |
|  | g) | List out the properties of conducting materials | CO3 | L3 | 1M |
|  | h) | State Biot - Savart’s law. | CO3 | L3 | 1M |
|  | i) | State the Lorentz force equation for a moving charge. | CO4 | L3 | 1M |
|  | j) | Write the point form of Ampere’s circuital law. | CO3 | L2 | 1M |
|  | k) | Write Magnetic Boundary Conditions. | CO3 | L2 | 1M |
|  | l) | Define Displacement current and current density. | CO3 | L2 | 1M |
|  | m) | Write Maxwell’s I equation and II equation for time varying fields. | CO5 | L2 | 1M |
|  | n) | Define skin depth. | CO5 | L2 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Derive the expression for electric field intensity due to surface charge. | CO1 | L3 | 7M |
|  | b) | Four point charges each of 100μC are placed in free space at the points (1,0,0), (-1,0,0), (0,1,0) & (0,-1,0)m respectively. Calculate the force on a point charge of 30μC located at a point (0,0,1)m. | CO1 | L4 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | Apply Gauss’s law to calculate the E due to infinitely long conducting wire placed along Z axis. | CO1 | L3 | 7M |
|  | b) | Given the potential, V=(10/r2) sinθ cosφ, Find D at (2,π/2,0). | CO1 | L4 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | Obtain the expression for electric field intensity due to an electric dipole. | CO2 | L4 | 7M |
|  | b) | Derive the expression for capacitance of a parallel plate capacitor. | CO2 | L4 | 7M |
| (OR) |  |  |  |  |  |
| 5 | a) | Derive the equations for Dielectric - Dielectric boundary conditions. | CO2 | L3 | 7M |
|  | b) | Derive the equations for Dielectric – free space boundary conditions. | CO2 | L4 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | Apply Biot-Savart’s law to derive an expression for the magnetic field intensity in the vicinity of a straight current carrying conductor of infinite length. | CO3 | L3 | 7M |
|  | b) | Derive the expression for force on a differential current element placed in a magnetic field. | CO3 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | State and prove Ampere’s circuital law. | CO3 | L3 | 7M |
|  | b) | An infinitely long conducting filament is placed along Z-axis and carries a current of 10 mA in az direction. Find H at (-2, 3, 4). | CO3 | L4 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Write the Maxwell’s equations both in point and integral forms for time varying fields. | CO4 | L2 | 7M |
|  | b) | Derive the expression for intrinsic impedance when uniform plane wave propagating through lossy dielectric. | CO5 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | A point charge of Q = -1.2C has velocity v=(5ax+2ay−3az)ms. Find the magnitude of the force exerted on the charge if, i)E=(−18ax+5ay−10az)Vm,ii)B=(−4ax+4ay+3az)T. iii) Both are present simultaneously. | CO4 | L3 | 7M |
|  | b) | State and prove poynting theorem. | CO5 | L4 | 7M |