20EI303

Hall Ticket Number:

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2024 | Electronics and Instrumentation Engineering |  |  |
| Third Semester | Digital Electronics |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | Find the signed 2’s complement representation of ‘-25 ’ | CO1 | L1 | 1M |
|  | b) | Express the Boolean function F = AB +AC +A’D in a product of Maxterms form. | CO1 | L3 | 1M |
|  | c) | What is Excess-3 code? | CO1 | L1 | 1M |
|  | d) | What are essential prime implicants in tabulation method? | CO2 | L1 | 1M |
|  | e) | How to obtain two input EX-NOR gate from EX-OR gate? | CO2 | L3 | 1M |
|  | f) | How to convert a binary code to a Gray code? | CO2 | L4 | 1M |
|  | g) | List the applications of Encoder. | CO3 | L1 | 1M |
|  | h) | How to implement demultiplexer from decoder? | CO3 | L4 | 1M |
|  | i) | Draw the excitation table of a T Flip Flop. | CO4 | L1 | 1M |
|  | j) | How to set and reset the JK Flip Flop using asynchronous inputs. | CO4 | L3 | 1M |
|  | k) | Differentiate combinational and sequential circuit. | CO1 | L4 | 1M |
|  | l) | What is shift Register? | CO1 | L1 | 1M |
|  | m) | What is Johnson counter? | CO1 | L2 | 1M |
|  | n) | Give the advantages of CMOS logic family over TTL. | CO2 | L2 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Perform the following subtraction using 1’s complement method. i) 01010-01001 ii) 01100-01100.11 | CO1 | L5 | 7M |
|  | b) | Find the value of x in the following equations. i) 1110x = 9B16 ii) FADE16 = x10 | CO1 | L3 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | Convert the following numbers as indicated: i)(62.7)8 = ( )16 = ( )2 ii) (BC64)16 = ( )10 = ( )2 | CO1 | L3 | 7M |
|  | b) | State duality theorem. List Boolean laws and their duals. | CO1 | L1 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | Simplify the following Boolean function for minimal SOP form using K-map and implement using NAND gates. F(W,X,Y,Z) = Σ(1,3,5,8,9,11,15) + D(2, 13) | CO2 | L4 | 7M |
|  | b) | Explain the operation of a full adder circuit with the help of truth table. Implement it using two half adders. | CO2 | L2 | 7M |
| (OR) |  |  |  |  |  |
| 5 | a) | What are the steps to be taken for gray to binary conversion? | CO2 | L1 | 7M |
|  | b) | Use the tabulation procedure to generate the set of prime implicants and to obtain all minimal expression for the following function f(A,B,C,D)=∑(0,1,4,5,6,7,9,11,15) + ∑ d(10,14) | CO2 | L6 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | Implement the logic expression f(A,B,C,D)=∑m(0,2,4,6,8,10,12,13,15) using a i) 4:1 multiplexer ii) 8:1 multiplexer | CO3 | L3 | 7M |
|  | b) | What is race around condition? How it is avoided in master-slave JK flip flop? Explain with necessary diagrams. | CO3 | L1 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | A combinational circuit is defined by the following three Boolean functions F1 = x’y’z’+xz F2= xy’z’+x’y F3= x’y’z+xy Design the circuit with a decoder and external gates. | CO3 | L6 | 7M |
|  | b) | Draw and explain the operation of a clocked SR Flipflop. What are its limitations. | CO3 | L2 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Design a Mod-6counter circuit using JK flip flops. | CO4 | L6 | 7M |
|  | b) | Explain the operation of a ring counter. | CO4 | L2 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | Draw and explain the NOR gate using CMOS logic and explain the operation. | CO4 | L2 | 7M |
|  | b) | Compare PLA, PAL, and PROM with block diagrams. | CO4 | L2 | 7M |