Hall Ticket Number:                                                                                                       20EI302

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2024 | Electronics & Instrumentation Engineering |  |  |
| Third Semester | Electronic Devices and Circuits |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | What is meant by doping in semiconductor? | CO1 | L1 | 1M |
|  | b) | Which charge carriers of majority and minority carrier in N-type Semiconductor? | CO1 | L2 | 1M |
|  | c) | Give the expression for the Fermi level energy in N- type semiconductor. | CO1 | L2 | 1M |
|  | d) | What is break down? What are its types? | CO2 | L1 | 1M |
|  | e) | Write BJT specifications. | CO2 | L2 | 1M |
|  | f) | Define reverse leakage current in C.E configuration. | CO2 | L2 | 1M |
|  | g) | Draw the symbols of Depletion MOSFET (n- channel/p-channel) | CO3 | L1 | 1M |
|  | h) | What are the terminals of a MOSFET. | CO3 | L3 | 1M |
|  | i) | What are the advantages of self-biasing circuit? | CO4 | L2 | 1M |
|  | j) | Draw I-V characteristic of a diode. | CO2 | L1 | 1M |
|  | k) | What is forbidden energy gap? | CO1 | L2 | 1M |
|  | l) | What is mobility? What is its unit? | CO1 | L1 | 1M |
|  | m) | Define mean life time of a hole and electron. | CO1 | L1 | 1M |
|  | n) | What is meant by Q-point? | CO2 | L2 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Define Hall effect and Derive Hall coefficient expression along with applications of Hall effect | CO1 | L2 | 7M |
|  | b) | Derive the expression for Fermi Level in an intrinsic semiconductor. | CO1 | L3 | 7M |
|  |  | (OR) |  |  |  |
| 3 |  | Explain the Diffusion and Drift currents for a semiconductor. | CO1 | L2 | 14M |
| Unit-II |  |  |  |  |  |
| 4 | a) | Explain the formation of depletion region in a PN junction. | CO2 | L2 | 7M |
|  | b) | Explain about input and output characteristics of a transistor when it is connected in common base configuration. | CO2 | L2 | 7M |
| (OR) |  |  |  |  |  |
| 5 | a) | Explain input and output characteristics of common emitter configuration. | CO2 | L2 | 7M |
|  | b) | A PN junction silicon diode has a reverse saturation current of 50nA at room temperature 270C. If the new reverse saturation current is observed to be 160nA, calculate the value of new temperature. | CO2 | L3 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | Explain the Vds/Ids characteristics and operation of depletion type MOSFET. With suitable diagram. | CO3 | L2 | 7M |
|  | b) | Draw the small signal model of the MOSFET and write expressions for trans conductance and output conductance | CO3 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | Discuss the operation of enhancement MOSFET at different regions with suitable figures and equations. | CO3 | L2 | 7M |
|  | b) | Explain the Small signal operation of CS amplifier. Find the expressions for input resistance (Rin), Output resistance (Rout), voltage gain (Av). | CO3 | L3 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Draw and explain active current mirrors circuit. | CO4 | L2 | 7M |
|  | b) | A MOS differential pair is driven with an input CM level of 1.6V. If ISS = 0.5mA, VTH = 0.5V, and VDD = 1.8V, what is the maximum allowable load resistance? | CO4 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | Draw and explain CG biasing circuit. | CO4 | L2 | 7M |
|  | b) | Write about single ended and differential operation | CO4 | L3 | 7M |