Hall Ticket Number:                                                                                          20EI304

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2024 | Electronics and Instrumentation Engineering |  |  |
| Third Semester | Network Theory |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | Define active elements with suitable example | CO1 | L1 | 1M |
|  | b) | Explain source transformation with an example | CO1 | L1 | 1M |
|  | c) | For the given circuit write the KVL equations | CO1 | L1 | 1M |
|  | d) | Define form factor | CO2 | L1 | 1M |
|  | e) | Find the r.m.s value of the voltage wave whose equation is, v(t)=10+200 sin(ωt-300 )+100 cos3ωt -50 sin (ωt+600 ) | CO2 | L1 | 1M |
|  | f) | State superposition theorem | CO2 | L1 | 1M |
|  | g) | Define reactive power. | CO2 | L1 | 1M |
|  | h) | Draw the steady state DC response of current in RL circuit. | CO3 | L1 | 1M |
|  | i) | Define time constant | CO3 | L1 | 1M |
|  | j) | What is meant by transient response | CO3 | L1 | 1M |
|  | k) | Find the condition for critical damping in a R-L-C series circuit | CO3 | L1 | 1M |
|  | l) | Write the Laplace transform of impulse signal | CO4 | L1 | 1M |
|  | m) | State final value theorem of Laplace transforms. | CO4 | L1 | 1M |
|  | n) | What is the condition for symmetry in transmission line parameters | CO4 | L1 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Use Ohm’s law and Kirchhoff’s laws on the circuit given in Fig., find Vin, Vs and power provided by the dependent source. | CO1 | L3 | 7M |
|  | b) | Calculate equivalent resistance between the terminals shown in fig below | CO1 | L3 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | Determine vo and i in the circuit shown in Fig. P.T.O | CO1 | L4 | 7M 20EI304 |
|  | b) | Derive the expressions to convert the delta network in to an equivalent star network and convert the following delta network in to an equivalent star network. | CO1 | L3 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | A 10 V rms, 10 kHz signal is applied to a series RL circuit consisting of a 1kΩ resistor connected in series with a 50mH coil. Find impedance Z, current I, phase angle θ. | CO2 | L4 | 7M |
|  | b) | For a load, Vrms = and Irms = Determine: (a) the complex and apparent powers, (b) the real and reactive powers, and (c) the power factor and the load impedance. | CO2 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 5 | a) | State and explain maximum power transfer theorem and find the maximum power transferred to resistor R in the circuit of Fig. | CO2 | L3 | 7M |
|  | b) | State and explain Thevenin’s theorem and Reciprocity theorem for any network. | CO2 | L4 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | For RLC series resonance circuit derive the expressions for fo , Band width and Q factor. | CO3 | L4 | 7M |
|  | b) | Differentiate the series and parallel resonance. | CO3 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | What is the condition for the response of a series RLC circuit excited by DC supply to have critically damped response? | CO3 | L3 | 7M |
|  | b) | Derive an expression for i(t) of a series R-L circuit excited by a sinusoidal source. | CO3 | L3 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Derive the expression for step response of series R-C circuit. Use Laplace transform approach | CO4 | L4 | 7M |
|  | b) | Find the inverse Laplace transform of the following signal | CO4 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | Express the Z-parameters of a two-port network in terms of Y parameters. | CO4 | L4 | 7M |
|  | b) | Determine the Y- parameters of the network shown | CO4 | L3 | 7M |