Hall Ticket Number:                                                                                       20ME305

| III/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2024 | Mechanical Engineering |  |  |
| Third Semester | Fluid Mechanics &Hydraulic Machinery |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | Define viscocity? | CO1 | L1 | 1M |
|  | b) | What is surface tension? | CO1 | L1 | 1M |
|  | c) | State Pascals law. | CO1 | L1 | 1M |
|  | d) | What is centre of pressure? | CO2 | L1 | 1M |
|  | e) | State assumptions in Bernoullis equation. | CO2 | L1 | 1M |
|  | f) | State Impulse momentum principle. | CO2 | L1 | 1M |
|  | g) | Give the principle of Reynolds experiment. | CO3 | L2 | 1M |
|  | h) | What is water hammer? | CO3 | L1 | 1M |
|  | i) | Give the expression for power transmitted through a pipe. | CO4 | L1 | 1M |
|  | j) | Explain angular momentum principle? | CO4 | L2 | 1M |
|  | k) | Give the example for mixed flow turbine? | CO1 | L2 | 1M |
|  | l) | What is the function of draft tube? | CO1 | L1 | 1M |
|  | m) | What is cavitation? | CO1 | L1 | 1M |
|  | n) | Why the Air vessels are connected to the reciprocating pumps? | CO2 | L2 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Differentiate between simple and differential manometers. | CO1 | L2 | 7M |
|  | b) | A plate having an area of 0.7 m 2 is sliding down the inclined plane at 450 to the horizontal with a velocity of 0.45 m/s. There is a cushion of fluid 2 mm thick between the plane and the plate. Find the viscosity of the fluid if the sweight of the plate is 300 N. | CO1 | L3 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | Describe the different types of fluid flows and examples. | CO1 | L2 | 7M |
|  | b) | In a two-dimensional incompressible flow, the fluid velocity components are given by u = x – 4y and v = -y -4x. Show that velocity potential exists and determine its form. Find also the stream function. | CO1 | L3 | 7M |
| Unit-II |  |  |  |  |  |
| 4 |  | Starting from the first principles, derive the Eulers equation of motion. | CO2 | L3 | 14M |
| (OR) |  |  |  |  |  |
| 5 | a) | Derive the Darcy-Wiesbach equation. | CO2 | L3 | 7M |
|  | b) | 400 lit/s of water is flowing in a pipe. The pipe is bent by 120o. The pipe bend measures 360 mm x 240 mm and volume of the bend is 0.15 m3. The pressure at the entrance is 80 kN/m2 and the exit is 1.8 m above the entrance section. Find the force exerted on the pipe. | CO2 | L4 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | Derive the expression for force and power when the jet is striking at the center of vertical stationary plate and moving plate. | CO3 | L2 | 7M |
|  | b) | A jet of water having a velocity of 36m/sec strikes a series of radial curved vanes connected on a wheel which is rotating at 240rpm. The jet makes an angle of 200 with the tangent to the wheel at inlet and leaves the wheel with a velocity of 6m/sec at an angle of 1300 to the tangent to the wheel at outlet. Water is flowing from outward in a radial direction. The outer and inner radiuses of the wheel are 500mm and 250mm respectively. Determine i) Vane angle at inlet and outlet and ii) Efficiency of the wheel. | CO3 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | Discuss the various heads and efficiencies in Hydraulic turbines. | CO3 | L2 | 7M |
|  | b) | A Pelton wheel has a mean bucket speed of 35 m/s with a jet of water flown at the rate of 1 m3/s under a head of 270 m. The buckets deflect the jet through an angle of 170°. Calculate the power delivered to the runner and hydraulic efficiency of the turbine. Assume coefficient of velocity as 0.98. | CO3 | L3 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Explain the working of a single stage Centrifugal Pump with neat sketch. | CO4 | L2 | 7M |
|  | b) | The impeller of a centrifugal pump having external and internal diameters 500mm and 250mm respectively, width at outlet 50mm and running at 1200rpm works against a head of 48m. Velocity of flow through the impeller is constant and equal to 3m/sec. The vanes are set back at an angle of 400 at outlet. Determine: (i)Inlet vane angle and (ii) Work done/sec | CO4 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | Define slip and negative slip in reciprocating pump. | CO4 | L2 | 4M |
|  | b) | Describe the single acting and double acting reciprocating pump with neat sketches. | CO4 | L2 | 10M |