20ME303

Hall Ticket Number:

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2024 | Mechanical Engineering |  |  |
| Third Semester | Strength of Materials |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | How fixed beam can be made statically determinate? | CO1 | L1 | 1M |
|  | b) | Define Tensile Stress and Compressive Stress of a material | CO1 | L1 | 1M |
|  | c) | How do you classify loads? Give examples. | CO1 | L1 | 1M |
|  | d) | Differentiate Beam and Bar | CO1 | L1 | 1M |
|  | e) | What do you mean by simple bending or pure bending and strength of the section? | CO2 | L1 | 1M |
|  | f) | Write the relation between bending moment, shear force and the applied load | CO2 | L1 | 1M |
|  | g) | Write down any two assumptions in the derivation of Torsion equation? | CO2 | L1 | 1M |
|  | h) | What do you mean by strength of a shaft and define polar modulus? | CO2 | L1 | 1M |
|  | i) | Define deflection and slope of a beam. | CO3 | L1 | 1M |
|  | j) | Show the location of neutral axis for a symmetric I-section with help of neat sketch. | CO3 | L1 | 1M |
|  | k) | Define the Section modulus | CO3 | L1 | 1M |
|  | l) | What is mohr’s circle? | CO4 | L1 | 1M |
|  | m) | Mention any two assumptions made in the Euler’s column theory | CO4 | L1 | 1M |
|  | n) | What is equivalent length of a column? | CO4 | L1 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | A member ABCD is subjected to point loads P1, P2, P3 andP4 as shown in figure Calculate the force P3, necessary for equilibrium if P1 = 120 kN, P2 = 220 kN and P4 = 160 kN. Determine also the net change in length of the member. Take E = 200 GN/m | CO1 | L3 | 10M |
|  | b) | Derive the expression for maximum stress induced and elongation produced in the bar due to its own weight. | CO1 | L3 | 4M |
|  |  | (OR) |  |  |  |
| 3 |  | A 10 mm diameter tensile specimen has a 50 mm gauge length. The load corresponding to the 0.2% offset is 55 kN and the maximum load is 70 kN. Fracture occurs at 60 kN. The diameter after fracture is 8 mm and the gauge length at fracture is 65 mm. Calculate the following properties of the material from the tension test. (i) % Elongation (ii) Reduction of Area (RA) % (iii) Tensile strength or ultimate tensile strength (UTS) (iv) Yield strength (v) Fracture strength (vi) If E = 200 GPa, the elastic recoverable strain at maximum load (vii) If the elongation at maximum load (the uniform elongation) is 20%, what is the plastic strain at maximum load? | CO1 | L3 | 14M |
| P.T.O 20ME303 |  |  |  |  |  |
| Unit-II |  |  |  |  |  |
| 4 |  | Two shafts of the same material and of same lengths are subjected to the same torque, if the first shaft is of a solid circular section and the second shaft is of hollow circular section, whose internal diameter is 2/3 of outside diameter and the maximum, shear stress developed in each shaft is the same, compare the weights of the shafts. Also find the percentage saving in weight. | CO2 | L3 | 14M |
| (OR) |  |  |  |  |  |
| 5 |  | Simply supported beam of length of 10m, carries the UDL and two point loads as shown in below figure. Draw SF and BM diagrams and also find out the Maximum values. | CO2 | L3 | 14M |
| Unit-III |  |  |  |  |  |
| 6 | a) | List out the assumptions of simple bending | CO3 | L2 | 4M |
|  | b) | From fundamentals derive the bending equation used in theory of simple bending | CO3 | L3 | 10M |
| (OR) |  |  |  |  |  |
| 7 | a) | What is moment area method? Explain the two Mohr’s theorems, as applicable to the slope and deflection of a beam. | CO3 | L3 | 7M |
|  | b) | A cantilever of length 3m is carrying a point load of 25kN at the free end with the moment of inertia of the beam is 108 mm4 and the value of E = 2.1x105 N/mm2. Find (i) Slope of the cantilever at free end and (ii) Deflection at the free end. | CO3 | L3 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 |  | State the equations for normal and tangential stresses on an inclined plane, in an element under general two-dimensional stress system. Derive the expressions for principal planes, principal stresses and maximum shear stress. | CO4 | L3 | 14M |
| (OR) |  |  |  |  |  |
| 9 |  | A hollow cast iron column 200 mm outside diameter and 150 mm in side diameter, 8 m long has both ends fixed. It is subjected to a Compressive load. Take a factor of safety of 6, σc = 560 N/mm2 and α =1/1600, determine the safe Rankine load | CO4 | L3 | 14M |