Hall Ticket Number:                                                                                            20ME304

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2024 | Mechanical Engineering |  |  |
| Third Semester | Engineering Thermodynamics |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | What is the Zeroth law of thermodynamics? | CO1 | L1 | 1M |
|  | b) | What is the work done during free expansion? | CO1 | L1 | 1M |
|  | c) | What is Quasi-static process? | CO1 | L1 | 1M |
|  | d) | What is perpetual motion machine of the second kind (PMM-2)? | CO2 | L2 | 1M |
|  | e) | What are limitations of first law of Thermodynamics? | CO2 | L2 | 1M |
|  | f) | Write the difference between steady flow and non-flow process. | CO2 | L1 | 1M |
|  | g) | What do you understand by availability? | CO3 | L2 | 1M |
|  | h) | Define entropy. | CO3 | L1 | 1M |
|  | i) | Give the order of efficiencies of Otto, dual, diesel cycles for same maximum temperature. | CO4 | L2 | 1M |
|  | j) | Draw the P-V and T-S plots of Otto cycle. | CO4 | L1 | 1M |
|  | k) | Write down the characteristic gas equation. | CO1 | L1 | 1M |
|  | l) | What is thermodynamic cycle? | CO1 | L1 | 1M |
|  | m) | Why is the performance of a heat pump or a refrigerator not measured in terms of thermal efficiency, but in terms of COP? | CO2 | L3 | 1M |
|  | n) | Write the expression for COP of a heat pump. | CO2 | L1 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Define thermodynamic work. Write similarities & dissimilarities between Heat and Work. | CO1 | L2 | 7M |
|  | b) | The cylinder of a gas engine has a volume of 0.001m3 and contains air at a pressure of 110kPa and temperature 21 °C. Calculate the mass of the air contained in the cylinder. Assume R = 0.287kJ/kg.K. | CO1 | L3 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | Explain i) Thermodynamic equilibrium. ii) Quasi-static process . | CO1 | L2 | 7M |
|  | b) | A Piston and cylinder machine containing a fluid system has a stirring device in the cylinder. The piston is frictionless, and it is held down against the fluid due to atmospheric pressure of 101.325 kpa. The stirring device is turned 10,000 revolutions with an average torque against the fluid of 1.25 mN. Meanwhile the piston of 0.6 m diameter moves out 0.8 m. Find the network transfer for the system. | CO1 | L3 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | Show that internal energy is a property of the system. | CO2 | L2 | 7M |
|  | b) | What is a flow process and derive the steady flow energy equation by applying first law of thermodynamics for a flow process? | CO2 | L2 | 7M |
| (OR) |  |  |  |  |  |
| 5 | a) | Show that the Kelvin - Planck and Clausiv's statement of the second law of thermodynamic are equivalent. | CO2 | L2 | 7M |
| P.T.O 20ME304 |  |  |  |  |  |
|  | b) | A reversible heat engine is supplied with heat from two constant temperature sources at 900 K and 600 K and rejects energy to constant temperature sink at 300 K. Assuming the engine to execute a number of complete cycles while developing 100 KW and rejecting 3200 KW of heat per minute, calculate the heat supplied by each source and efficiency of the engine. | CO2 | L4 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | State and explain principle of increase of entropy. | CO3 | L2 | 7M |
|  | b) | Two copper blocks weighing 10kg each are initially at temperatures of 227 °C and 27 °C respectively. What is the change in entropy when these two blocks are brought in contact with each other? Assume specific heat of copper as 0.4KJ/kg k. | CO3 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | Define Gibb's and Helmholtz functions and explain its significances | CO3 | L2 | 7M |
|  | b) | One kg of ice at -20 °C is exposed to the atmosphere which is at 20 °C. The ice melts and comes into thermal equilibrium with the atmosphere. Calculate the entropy increase of the universe. Take Cp of ice as 2.093 kJ/kg K and latent heat of the fusion of ice as 334.5 kJ/kg. | CO3 | L3 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Show that the efficiency of the Otto cycle depends only on the compression ratio. | CO4 | L3 | 7M |
|  | b) | Compare Otto, Diesel and Dual cycles operating at same compression ratio, same maximum temperature and same maximum pressure. | CO4 | L2 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | Write the differences between SI and CI internal combustion engines | CO4 | L2 | 7M |
|  | b) | Explain the working principle of four stroke Diesel engine with neat sketch | CO4 | L1 | 7M |